MOSFLM 6.0 User Guide

This describes MOSFLM version 6 for processing image plate and CCD data

(29 June 1999)

Andrew G.W. Leslie

MRC Laboratory of Molecular Biology

Hills Road,

Cambridge CB2 2QH

UK

E-mail: andrew@mrc-lmb.cam.ac.uk

Tel (+44) (0) 1223-248011

Any constructive comments on this User Guide would be very welcome.

Index

Major Changes

Help Library

Important notes

1:  Overview

1.1     Programs covered in this guide

1.2     Input and Output files

1.3     Allowed detector types

1.3.1   Using the DETECTOR keyword

1.3.2   Using the SITE keyword

1.3     Allowed detector types

1.4     Inspection of images

2:  A Quick Guide

2.1     Startup keywords

2.2     Autoindexing

2.3     Estimating mosaic spread

2.4     Running the Strategy option

2.5     Determining oscillation angles with

the TESTGEN option

2.6     Integrating the first image to determine

if the exposure time is OK

2.7     Interpreting those "WARNING" messages

2.8     Getting accurate cell parameters

2.9     Integrating a block of images

2.10    Integrating the dataset

3:  Determination of crystal orientation, cell parameters and spacegroup

3.1     Autoindexing Interactively

3.2     Autoindexing when running the program in background

4:  Running the STRATEGY and TESTGEN options

4.1     Overview of the STRATEGY option

4.2     Some Examples of the STRATEGY options

4.3     Determining the oscillation angle for each

image (TESTGEN option)

5:  Determining Accurate Cell parameters

5.1     Using Post-refinement to refine the cell

6:  Collecting data and processing the images

6.1     Overview

6.2     Special MOSFLM features

6.2.1   Accumulating profiles over several images

6.2.2   Addition of partials (ADDPART)

6.2.3   Post-refinement of orientation and cell

parameters

6.2.4   Optimisation of measurement box parameters

6.3     Running a processing job

6.3.1   Running MOSFLM interactively

6.3.2   Processing the first block of data)

(Non-interactively)

6.3.3   Finally, Processing the dataset

7:  Interpreting the output

7.1     The log files

7.2     The summary file

7.3     Checking the quality of the data

8:  General tips.

8.1     Estimating the GAIN of a detector

8.2     Processing images with no (or very few) fully

recorded reflections

8.3     Processing images when the spots are not fully resolved

8.4     Processing data from other detectors, or standard

detectors with different rotation axis orientation.

9:   Example command files

9.1     Autoindexing an initial image (interactively)

9.2     Determining an accurate cell

9.3     Integrating a series of images

Appendix I

Changes in MOSFLM

Appendix II

Setting the measurement box parameters manually

Appendix III

Overview of the MOSFLM program

Appendix IV

Definition of coordinate systems

----------------------------------------------------------------------------

MAJOR CHANGES

Changes since 6.00

Cell refinement added to the new DPS autoindexing, plus refinement of direct

beam position. Manual spot-finding option. Option to write multiple MTZ

files (if processing while collecting data). Fit circles option. Bug fix for

non-zero two-theta indexing. Many small bug fixes.

Changes since 5.51

New FFT based autoindexing algorithm from DPS added (Steller,Bolotovsky and

Rossmann (1998) J. Appl. Cryst. 30, 1036-1040.) New detector types LIPS

(large image plate scanner at ESRF) and SBC1 (Westbrook detector at APS)

added.

Changes since 5.50

New detector type MARCCD for 135mm circular CCD detector from Mar Research.

Allows partials to lie on up to 100 images (previous limit was 10) New

keyword TEMPLATE for more general definition of image filenames. Generalise

direction of two-theta axis (previously had to be parallel to fast changing

direction in image). More changes to STRATEGY algorithm. Standardise FORTRAN

so thet code will compile under Linux (this needs a new version of the

autoindexing code...post 5/5/98)

Changes since 5.40

Improved strategy algorithms. New detector types (Mar345, ADSC CCD) A host

of minor bug fixes, and changes to make the program easier to run.

Changes since 5.30

The major change to version 5.40 is that the code for the spot-finding

program (IMSTILLS) and autoindexing (REFIX) has been incorporated into

MOSFLM. A new menu for the X-window interface has been introduced, which

allows the user to find spots, autoindex images, run the strategy

option,refine cell parameters (using post-refinement) and integrate images

interactively. (All these features are of course still available when

running a background job). The new menu is invoked by using the "IMAGE"

keyword to read in an intial image. The data collection strategy option has

been available since version 5.30, but has been improved in the current

version, particularly by the option to speed up the calculation by any

desired factor and optimise anomalous data. Additional image formats have

been added. The program can now handle images from Mar Research, Raxis (II

or IV), Mac Science, Molecular Dynamics, Fuji and ESRF CCD detectors.

The following keywords are no longer necessary (but can still be given to

override program defaults) :

RASTER

SEPARATION

GENFILE

HKLOUT

PIXEL

and for Mar Research, ADSC, Mac Science and RaxisIV detectors:

WAVELENGTH

DISTANCE

and oscillation angles.

(Note however that the header information is not always correct for Mar

detectors at synchrotron sites, because the software controlling the spindle

axis (and/or distance) does not communicate with the Mar software

controlling the detector. Check this with the station manager)

See Appendix I for a more detailed list of major changes from earlier

versions.

----------------------------------------------------------------------------

Help Library

This User Guide is not exhaustive in describing all options available.

However all possible keywords are described in the help library file

(mosflm.hlp) which is part of the distribution. The help library is an ascii

file, and can therefore be read with an editor, or invoked by typing "HELP"

at the mosflm prompt (==>) when runing the program interactively. Note that

the environment variable "CCP4_HELPDIR" should point to the directory

containing the help library. The bugs that prevented the online help working

correctly have now been fixed.

----------------------------------------------------------------------------

Important notes

The source code as distributed contains the code for the FFT based

autoindexing but NOT for REFIX. I can distribute the REFIX code by E-mail to

academic institutions only, or to those who already have the Mar XDS

software. Please send an E-mail to the address given above to get the REFIX

autoindexing code.

----------------------------------------------------------------------------

Back to Contents Page

1: Overview

1.1     Programs covered in this guide

1.2     Input and Output files

1.3     Allowed detector types

1.3.1   Using the DETECTOR keyword

1.3.2   Using the SITE keyword

1.3     Allowed detector types

1.4     Inspection of images

1.5     Example input

----------------------------------------------------------------------------

1.1 Programs covered in this guide

Data processing falls naturally into three sections:

1) Determining the crystal orientation, cell parameters and

possible space group.

2) Generating the reflection lists and integrating the images.

3) Scaling and merging the resulting data.

These notes will be restricted to topics (1) and (2), which are now both

present in the MOSFLM program alone. The CCP4 program SCALA is strongly

recommended for the third step (scaling and merging).

1.2 Input and Output files

There are several input and output files and it is crucial that the output

files are given unique filenames when two (or more) processing jobs are

being run from the same directory, or the results are very unpredictable!

Input files

1) The image file

2) [The file containing the crystal orientation matrices]

NAMING CONVENTION FOR IMAGES

It is assumed that the images conform to a naming convention where the image

name is made up of three parts, a template, a three digit number and an

extension. The template can be up to 40 characters long, and should be

separated from the three digit number by a hyphen (-) or an underscore (_).

The extension can be up to 8 characters long and should be separated from

the three digit number by a period (.). Note that the template can contain

underscores or hyphens.

Examples of valid image filenames are:

lysozyme_cryst1_021.image

catx1_001.img

f1_tray42_wellb6_001.osc

ALTERNATIVE NAMING CONVENTION

If the image filenames do not conform to the specification given above, the

TEMPLATE keyword can be used to define a very general format for the image

filenames. If the TEMPLATE keyword is to be used, it MUST preceed the IMAGE

keyword in the input, and the image NUMBER, not the filename, should be

given. See TEMPLATE for more information.

eg

TEMPLATE fred_###

IMAGE 23 PHI 22 TO 23

will read the file "fred_023" (no filename extension).

Output Files

1) The output MTZ file containing the integrated intensities. Set with

keyword HKLOUT or on the command line using HKLOUT. When not given, a

default filename made up from the crystal identifier (or TEMPLATE) and the

first image number is used. It is now possible to specify that multiple MTZ

files are written during one integration run (subkeyword MULTIPLE on HKLOUT

keyword. In this case, a separate MTZ file will be written for each block of

images processed (see BLOCK subkeyword of PROCESS keyword). The filenames

will be distinguished by _001, _002, _003 etc being appended to the

specified (or default) filename. Eg lysx1_1to50_001.mtz, lysx1_1to50_002.mtz

etc. This allows some of the data to be scaled and merged prior to the data

processing finishing. (Binary)

2) If autoindexing or refining cell parameters a file containing the refined

crystal orientation matrices is written. Filename set with keyword NEWMAT,

defaults to NEWMAT. (Ascii)

3) The summary file. Contains a summary of processing results. Can be

assigned on the command line (SUMMARY), defaults to SUMMARY. This file can

be input to xloggraph for graphical representation.(Ascii)

4) When running interactively, all output written to the terminal window is

also written to the file "mosflm.lp". (Ascii)

Temporary Files

1) The "Generate" file. Assigned with keyword GENFILE, defaults to the same

as the MTZ filename but with the extension ".gen" instead of ".mtz".

(Binary)

2) The measurement boxes file. Assigned on the command line using SPOTOD

defaults to SPOTOD. Ths file can be very large, and should normally be

assigned to a scratch disk and deleted as part of the command procedure.

3) A reflection coordinate list, assigned using COORDS on the command line.

This is only produced when the "SEPARATION CLOSE" option is being used to

process images with very closely separated spots.

Example of a command line:

ipmosflm HKLOUT lyso_srs.mtz  SUMMARY lyso_srs.sum \

SPOTOD /scr0/andrew/lys.spotod \

COORDS /scr0/andrew/lys.coords  << eof-ipmos

GENFILE /scr0/andrew/lys.gen

NEWMAT lyso_srs.mat

....

....

eof-ipmos

1.3 Allowed detector types

The type of detector is specified either by the DETECTOR keyword, or by a

SITE keyword, with the latter generally being used for synchrotron sites

that use detectors that are not commercially available. At present, the

follow detectors are allowed.

1.3.1 Using the DETECTOR keyword

* Mar Research (18cm (SMALLMAR), 30cm or 34.5 cm plate (MAR));

* Mar Research CCD detector (MARCCD);

* ADSC Quantum 4 CCD detector (ADSC);

* Mac Science Dip2000 (DIP2000), 2020 (DIP2020) or 2030

(DIP2030) (horizontal or vertical rotation axis);

* Raxis II (RAXIS) (horizontal or vertical rotation axis),

* Raxis IV (RAXIS4 or RAXISIV) (Horizontal or vertical rotation

axis),

* ESRF Image intensifier CCD (ESRF CCD),

* Molecular dynamics (offline) (MD),

* FUJI scanners (offline) (FUJI BAS2000 or FUJI BA100),

* Ed Westbrook 3x3 CCD detector at APS (SBC1),

* ESRF Large Image Plate Scanner (LIPS).

Note that no special input is required to distinguish between the different

types of Mar Research image plate scanner (18,30 or 34.5cm (Mar345)). The

image size is read from the header record and the appropriate limits and

pixel size are set up automatically.

Both unpacked and packed image formats are supported for the Mar345 scanners

(no DETECTOR keyword required to distinguish these).

For Mar, Raxis and Mac Science scanners it is not necessary to specify the

size of the image, as it is determined from the image header.

For offline scanners (FUJI and MD) it will also be necessary to define the

orientation of the image relative to the X-ray beam and rotation axis, also

using the DETECTOR keyword. See the help library (Subsection Novel detectors

of DETECTOR) for details on how to do this.

If the rotation axis is reversed (usually a peculiarity of synchrotron

sites) this can be dealt with by specifying: DETECTOR REVERSEPHI ..again,

see the help library.

1.3.2 Using the SITE keyword

For CHESS, the station (A1, F1 or F2) and the detector must be specified.

Possible detectors are the Gruner CCD detector working in 1K, 2K or 2K

binned modes, the ADSC single module CCD detector (ADSC) the ADSC 2x2 CCD

detector (QUANTUM4) and FUJI image plates. eg

SITE CHESS [A1 F1 F2] [FUJI [CCD [1K 2K 2KBINNED ADSC QUANTUM4]]]

For SSRL and ALS, the 2x2 ADSC detector is allowed: eg

SITE SSRL ADSC,  SITE ALS ADSC

1.4 Inspection of images, invoking the new menu

It cannot be emphasised strongly enough that images must be examined closely

to check the following:

1) Does the crystal diffract ?

2) What is the effective resolution limit...should the detector be

moved further back to take advantage of the full active area of

the detector.

3) Is the crystal twinned, split, disordered etc ?

4) Is the exposure time long enough ?

Images can be displayed using the IMAGE keyword followed by the full

filename of the image (including the directory if the image is not in the

current directory). The only other keyword required specifies the type of

detector (default is Mar Research image plate scanners). This will bring up

the new menu interface which allows autoindexing, integration etc.

Note that MOSFLM displays the image viewed from the detector looking towards

the source (cameraman's view), and also that the "fast changing" direction

in the image is ALWAYS vertical in the display, regardless of whether it is

vertical or horizontal in the actual detector. Thus some images will be

rotated by 90 degrees.

example

IMAGE /fred/images/lysox1_001.image

GO

or

DETECTOR RAXISIV

IMAGE /fred/images/lysox1_001.image

GO

In order to measure the resolution of individual spots on the images or

display the resolution circles the wavelength and crystal to detector

distance need to be given. For Mar Research, ADSC, RaxisIV and Mac Science

detectors the wavelength and distance will automatically be taken from the

header records in the image file, for other types of image the DISTANCE and

WAVELENGTH keywords should be given, or the values set interactively using

the X-windows interface.

Parameters that may need to be defined (and the appropriate keywords) are:

1) crystal to detector distance (DISTANCE)

2) Wavelength (WAVE)

3) Direct beam coordinates (BEAM)

EXAMPLE INPUT

NOTE: Input within square brackets is optional for Mar, ADSC, RaxisIV and

Mac Science images. If a PHI keyword is given, this will override the phi

values in the image header, and phi values in the header will be ignored for

any subsequent image that is read in. The default DETECTOR type is MAR, so

this need not be given for Mar images.

IMAGE catx1_001.img [PHI 0.0 TO 1.0]

BEAM 149.5 151.0

DETECTOR MAR (or SMALLMAR, MARCCD, ADSC, RAXIS, RAXISIV, SBC1, DIP2000,

DIP2030, ESRF CCD, FUJI, MD)

[NEWMAT test_001.mat]   ! Defines the name of the file in which the results

! of autoindexing or postrefinement will be written.

[WAVELENGTH 1.542]

[DISTANCE 250.0]

GO

This will invoke the X-window display, and a Menu list as shown below:

Read image        Read in another image.

Find spots        Find spots on the current (displayed) image.

Edit spots        Allows manual rejection of spots.

Save spots        Writes all spots to a file for use with old REFIX.

Clear spots       Deletes spots from display or list of stored spots.

Select images     If spots have been found on several images, allows

selection of images to be used in autoindexing.

Autoindex         Invokes autoindexing (DPS or REFIX).

Predict           Predicts spot pattern.

Clear prediction  Deletes predicted pattern from display.

Adjust            Adjust the fit between observed and predicted patterns.

Refine cell       Invokes a POSTREF SEGMENT run to refine cell parameters.

Integrate         Allows integration of images.

Strategy          Run the strategy option.

Keyword input     Allows keyworded input.

Find hkl          Allows a specified reflection to be identified.

Pick              Display pixel values

Measure cell      Measure cell parameters.

Circles           Display resolution circles.

Fit circles       Allows fitting to circles in image.

Exit              Close down X-windows display.

The various options in this menu list are discussed in the sections below.

Note that there are some on/off and yes/no toggle boxes at the bottom of the

"Processing parameters" window. These are described below:

Prompts              On/Off (default on)

When "prompts" is "on", additional information is given when some

of the menu options are chosen. For experienced users, this

additional information can be suppressed by turning the prompts

"off".

Update display:

After refinement     No/Yes

After integration    No/Yes

By default, the display is updated each time a new image is read,

and at no other time. By setting the "After refinement" toggle to

Yes, the display will be updated after refinement of the detector

parameters, so that it is possible to check how well the predicted

pattern matches the image. If the "After integration" toggle is

set to yes, each image will be display after it has been

integrated, with "Bad spots" indicated and residual vectors

(beteen observed and predicted spot positions) for fully recorded

spots also shown. It is possible to reject additional reflections,

or reclassify Bad spots, at this point.

Note that because images are integrated in "Blocks", during the

actual integration of all images in a block the image that is

displayed will be that of the last image in the block, unless the

"After integration" toggle has been set to yes.

Timeout mode:        Off/On

If the Timeout mode is set to "On" during an Integration or Refine

Cell run, then when each image is displayed the program will wait

for 2 seconds for the user to select a menu option (it is best to

start by turning the Timeout mode off if you want to do this).

After this period (which can be changed with keyword "TIMEOUT")

the program will just carry on. With the timeout mode "On" it is

therefore possible to integrate a series of images without

directly interacting with the program. This can be very useful if

one just wants to keep an eye on the processing but do not want to

keep hitting the "Continue" menu option.

----------------------------------------------------------------------------

Back to Contents Page

2: A Quick Guide

2.1     Startup keywords

2.2     Autoindexing

2.3     Estimating mosaic spread

2.4     Running the Strategy option

2.5     Determining oscillation angles with

the TESTGEN option

2.6     Integrating the first image to determine

if the exposure time is OK

2.7     Interpreting those "WARNING" messages

2.8     Getting accurate cell parameters

2.9     Integrating a block of images

2.10    Integrating the dataset

----------------------------------------------------------------------------

This is a brief guide on how to process data using the new menu options. For

more details on each step in the procedure, see sections 3-6.

2.1 Startup keywords

Use the following keywords to bring up the image display and menu. These

should be given at the MOSFLM => prompt:

=> TITLE My lysozyme data               ! This title is transferred to the

! MTZ file

=> IMAGE lyso_001.image [PHI 0 TO 1]    ! Filename of first image. For Mar,

! ADSC, Mac Science and RaxisIV images

! the phi values will be taken from the

! image header if not given here.

! If phi values are specified here,

! the values in the header will be

! ignored for this an all subsequent

! images read in.

=> BEAM 150.0 149.0                     ! Direct beam coordinates

If not a Mar Research IP scanner:

=> DETECTOR RAXISIV                     ! or RAXIS (for RAXIS II) or DIP2000 etc

If not processing Mar Research, ADSC, R-axis or Mac Science images:

=> WAVE 0.91                            ! For Mar, ADSC, RaxisIV and Mac Science

=> DISTANCE 300                 ! this information is taken from the header

! but can be overwritten using the

! keywords.

=> SYMM p43212                  ! If known, give cell and symmetry

=> CELL 79 79 38                        ! otherwise omit completely.

Not essential for first stages, but needed for integration:

=> DIVERGENCE 0.1 0.03                  ! If isotropic, the beam divergence

! can be included in the mosaic spread.

=> SYNCHROTRON POLARIZATION 0.9         ! Defaults to 0.86 (SRS, Daresbury UK)

=> GAIN 1.7                             ! See section 8.1 for a way to

! estimate the gain if not known.

=> GO

At this point, the image will be displayed with a list of "Processing

parameters" on the far left (these can be changed by the user), a "Main

menu" and beneath the Main menu a table of "Output" parameters.

2.2 Autoindexing

Select the menu option "Autoindex". The program will locate and display

spots on the image. Parameters governing the spot finding are listed under

*SPOT SEARCH* in the Processing Parameters table, but the program

automatically sets suitable values for these parameters and they will not

normally have to be changed. The image will then be autoindexed. The user

will be prompted to supply a filename for the output orientation matrix. All

queries have a default, which can be selected by simply entering carriage

return.

When using the new DPS indexing, if a spacegroup and cell have been given,

the cell parameters determined by the autoindexing will be permuted to best

match the input values, but the user must still select the solution from the

list provided. If using REFIX, the same information will force the image to

be be autoindexed with this cell and no alternatives will be listed. If no

spacegroup information has been given, for both algorithms the user will be

presented with a list of choices, sorted on a "PENALTY" parameter (the lower

the PENALTY the better). The user must select a spacegroup, and the cell is

refined imposing that symmetry.

The success of the autoindexing can be checked by predicting the spots for

the current image using the menu option "Predict". If not successful, try

adjusting the intensity threshold "Min I/sig(I)" or the maximum cell length

(for the FFT based algorithm) or read in another image (Read image menu

option), find spots on it (Find spots) and repeat autoindexing (Autoindex).

Spots from a satellite crystal can be removed using the Edit spots option.

2.3 Estimate mosaic spread

An approximate estimate of the mosaic spread should be obtained by

predicting with different values of mosaic spread (set in Processing

parameters) and seeing what value gives the best fit to the observed

pattern.

2.4 Run Strategy option

Select the "Strategy" menu option. Input for this option has to be given at

the MOSFLM => prompt in the terminal window.

MOSFLM => STRATEGY

MOSFLM => GO

This will generate a reflection list, a unique reflections list, merge them

and tell you what rotation range to use to get a maximally complete dataset.

If you then want to reduce the total rotation range (to save time) and still

get a maximally complete dataset type the following at the STRATEGY =>

prompt:

STRATEGY => ROTATE 60 SEGMENTS 2

STRATEGY => GO

This instructs the program to find two 30 degree segements that give maximum

completeness. You can try 3 segments (of 20 degrees) if you like, but this

rarely (in my experience) gives significantly greater completeness and will

take significantly longer. (Also don't forget that the more segments you

have, the more unmatched partials you will get).

For orthorhombic space groups, you should also try STRATEGY ALTERNATE if the

predicted completeness in not as high as expected.

2.5 Determine oscillation angles with the TESTGEN option

Having determined what rotation range needs to be collected, you can check

what the (maximum) rotation angle is to avoid getting (too many) spatial

overlaps on the images. Remember you must have realistic estimates of the

mosaic spread and minimum spot separation for this to be meaningful. Also

remember that for post-refinement to work, the oscillation angle must be

more than half the sum of the mosaic spread and beam divergence.

At the STRATEGY => prompt type:

STRATEGY => TESTGEN

This will describe the possible keywords. If your data collection was in two

segments of -15 to 15 degrees and 45 to 75 degrees and you want no overlaps

type :

STRATEGY => TESTGEN START -15 END 15

STRATEGY => GO

and the program will calculate the MAXIMUM possible rotation angles for this

range, at intervals of 5 degrees.

then type:

STRATEGY => TESTGEN START 45 END 75

STRATEGY => GO

for the second segment.

2.6 Integrate the first image to determine if the exposure time is OK

The best way to get an indication of the data quality is to integrate the

first image and see how the mean <I>/sigma<I> varies with resolution. These

values are always slightly optimistic, so you should aim to have a ratio of

at least 3.0 in the outermost resolution bin. If it is lower than this,

consider collecting data to a lower resolution (and moving the detector

further back) or using a longer exposure time.

To get back to the Menu, type EXIT at the STRATEGY prompt:

STRATEGY => EXIT

Then select the "Integrate" menu option, and answer the questions (most can

be answered by entering carriage return). However, now is the time to set

the centre and radius of the backstop shadow, using the BACKSTOP keyword,

eg:

BACKSTOP CENTRE 88 90 RADIUS 12

An alternative way of dealing with backstop shadows (particularly for an

extended backstop) is to use the NULLPIX keyword. This is used to define a

minimum pixel value, and any spot that has a pixel within its measurement

box with a value lower than this minimum will be rejected.

The program will put up the predicted pattern and then wait for input

(unless the timeout mode is set). If the pattern is a good fit, choose menu

option "Continue". If the pattern is not aligned with the spots, choose the

option "Adjust" and follow the instructions to align the pattern with the

spots. (The usual reason for poor alignment is an error in the direct beam

coordinates).

The image will then be integrated. Check the <I>/sigma<I> values, and the

Rsym if you have symmetry related fully recorded reflections on this image.

The value of SDRATIO is a better guide than the actual Rsym, as the latter

will depend on the intensity of the reflections. The SDRATIO should lie in

the range 1 to 3.

2.7 Interpreting those "WARNING" messages

After the integration, the program will usually print a list of "WARNING"

messages. Don't worry about messages about the standard profiles at this

stage, or large positional residuals (because the cell has not yet be

accurately determined). However if the " OVERALL BACKGROUND RATIO (BGRATIO)"

message is present, this suggests the detector GAIN may be wrong, and the

input value need to be multiplied by the square of the value of the BGRATIO.

Beware, however, that images showing diffuse scatter will give a high

BGRATIO even when the GAIN is correct (eg up to 1.5).

2.8 Getting accurate cell parameters

Accurate cell parameters are essential for optimal integration of the

images. MOSFLM uses a post-refinement procedure to determine accurate cell

parameters. For trigonal or higher symmetry, an accurate cell can usually be

determined from a single "wedge" of data (typically 3-5 degrees), unless the

unique axis is approximately along the X-ray beam direction in which case

either a different phi value should be used or two segments of data. For

orthorhombic or lower symmetry two "wedges" of data widely separated in phi

will give the best results. In the latter case, one can either wait until a

large rotation range has been collected before refining the cell, or one can

start by collecting a few degrees at (say) phi 85 to 90, then start

collecting from phi = 0.

When the appropriate data (images) are available, select the "Refine cell"

menu option. Answer the queries. Although the default number of images to

use is 2 (in each wedge), this is in fact the minimum number and better

results will often be obtained by using 3 or 4 images in each wedge.

It is important to have a realistic estimate of the mosaic spread before

refining the cell.

Post-refinement yields very accurate cell parameters, but has a relatively

small radius of convergence. If the shift in cell parameters exceeds 2.5

sigma, the integration of the images and the actual refinement will be

repeated. This will happen up to a maximum of 5 times. It is not unusual for

2 or 3 complete rounds to be required if the initial cell parameter

estimates came from auto-indexing a single image.

2.9 Integrating a block of images

The next step is normally to integrate a block of between 5 and 10 images.

Use the "Integrate" menu option as before. In this case, pay particular

attention to the list of warning messages to see if any parameters or

options need to be reset. It is also a good idea to check the appearance of

the standard profiles (these are output to the terminal window but also to

the file "mosflm.lp"). Make sure that adjacent spots are being adequately

resolved, and that the peak is not spilling into those pixels marked as

background. The PROFILE TOLERANCE parameters are crucial in determining the

appearance of the standard profiles. Also try to ensure that NO profiles are

being averaged. If necessary, change the minimum rms variation in the

background (PROFILE RMSBG) or the number of different profiles (by defining

PROFILE XLINES and PROFILE YLINES) to avoid profile averaging. Check that

there are not too many reflections being rejected as "BAD SPOTS". If a

significant number of strong reflections are being rejected for "Poor

profile fit", if the cell has been accurately determined and the GAIN is

correct, consider increasing the rejection value (REJECTION PKRATIO) from

its default of 3.5 to 4.0. It should NOT be necessary to increase this above

4.0

2.10 Integrating the dataset

Once a block of images has been successfully integrated, the complete

dataset can be integrated. If data processing is started before data

collection is complete, use the WAIT keyword to make the program wait for an

image to be completed before it tries to integrate it.

eg WAIT 15 for 15 minute exposures.

You may also wish to specify multiple MTZ files (one for each "block" of

images) so that some data can be scaled and merged in SCALA before data

collection/processing has completely finished.

Set the "Timeout mode" toggle (at the bottom of the "Processing parameters"

window) so that the program will automatically continue 5 seconds after

displaying each new image.

Set the "Prompts" toggle to Off.

----------------------------------------------------------------------------

Back to Contents Page

3: Determination of crystal orientation, cell parameters and spacegroup

3.1     Autoindexing Interactively

3.2     Autoindexing when running the program in background

----------------------------------------------------------------------------

The crystal orientation, cell parameters and possible spacegroups are

normally determined from a single rotation image (although 2, 3 or more can

be used also). This will typically be a rotation of between 0.5 and 2

degrees, the value being chosen to avoid generating a significant number of

spatially overlapped reflections. Autoindexing can be performed

interactively or in batch mode, but spacegroup selection can ONLY be done

interactively as the user is required to select a cell and spacegroup from a

number of possibilities.

3.1 Autoindexing Interactively

3.1.1 Finding Spots

The first step in autoindexing an image is to locate the positions of

diffraction spots. This can be done with the "Find spots" menu option, but

if only one image is to be used for autoindexing one can go straight to the

"Autoindexing" menu option.

3.1.1.1 Parameters used in Finding Spots

Parameters associated with spotfinding are listed in the "Processing

parameters" window:

Threshold

Rmin

Rmax

X offset

Y offset

Min X size

Max X size

Min Y size

Max Y size

Min no of pix

X splitting

Y splitting

All of these parameters have "sensible" defaults and normally they do not

need to be changed.

Pixels are considered to be part of a spot if the pixel value is more than

(Threshold*sigma) above the local background at that radius. The threshold

is determined automatically by the program and will normally be appropriate.

The program searches for spots lying with radial limits of Rmin and Rmax

(mm) from the direct beam position. The first step is to determine a radial

background. The direction of this radial background is chosen to be at right

angles to the rotation axis (to avoid any backstop shadow). It is normally

centred on the direct beam position, but can be offset to one side by the "X

offset" or "Y offset" parameters. If the radial background is along Y (the

"fast" changing direction in the stored image), then use the "X offset" to

change its position. If the default direction is along Y, and a value is

entered for the "Y offset", this automatically changes the direction of the

background strip to be along the X axis. Entering a negative value for

either Rmin or Rmax will switch the background stripe to the opposite side

of the direct beam position.

The position of the background strip is shown as a red rectangle on the

display. If necessary its position should be changed to avoid any shadows or

other unusual features on the image.

Note that the LIMITS EXCLUDE keywords can be used to exclude rectangular

regions of the detector from spot finding (and integration).

The minimum and maximum spots sizes (in X and Y) are expressed as a multiple

of the median spot size. If the image is very strong and the threshold is

too low, then two adjacent strong spots may be treated as a single spot

(because the pixel values do not go down to the threshold inbetween them).

This problem can be avoided by either increasing the Threshold, or by

decreasing Max X and Y sizes, as these spots will be almost twice as large

as the average spot.

"Min no of pix" sets the minimum number of pixels that constitute a proper

spot.

Split spots will be treated as a single spot if they are less than "X

splitting" and "Y splitting" mm apart in X and Y. Normally this is not a

problem with image plate data.

In tricky cases (eg very weak spots on a high background) it may be

necessary to add spots manually. This is done by clicking on spots with the

mouse. You do not need to be exactly on the spot, the program will search

the area in the vicinity of the mouse and find the centre of gravity which

will be used as the spot position.

3.1.1.2 Displaying found spots

The positions of the spots found are displayed as red crosses. Note however

that ONLY spots which will be used for autoindexing (ie those with I/sig(I)

greater than the threshold) are displayed. This is determined by the only

parameter associated with autoindexing, the "Min I/sig(I)" parameter which

follows the spot finding parameters in the "Processing parameters" window.

This value defaults to 20.

It is a good idea to check that program is correctly locating the spots, and

that in particular if the spots are very close and the image is strong, it

is not treating two neighbouring strong spots as a single spot (in which

case the red cross will come half-way between the two spots). If this is a

problem, try increasing the "Threshold" or decreasing the "Max X size" and

"Max Y size".

3.1.1.3 Editing found spots

If the crystal is not single, and the program finds spots that do not lie on

the major lattice, it is a good idea to remove these spots. If the second

lattice is much weaker than the main lattice, it may be possible to do this

just by increasing the "Min I/sig(I)" parameter. If this does not work,

select the "Edit spots" option from the main menu. Identify spots to be

deleted by clicking on them with the mouse...this will result in an "X"

being written over that spot. Be careful, as the mouse must be quite close

to the spot position in order to reject the spot. When editing is finished,

click on the "End edit" in the main menu.

The autoindexing algorithm is quite sensitive to the presence of "rogue"

spots, so it is usually a good idea to reject them if the autoindexing is

not successful.

3.1.2 Finding spots on other images

If you want to use more than a single image for the autoindexing (and this

can provide successful autoindexing when using a single image fails) then

read in another image using the "Read image" option in the Main Menu. The

phi values will be read from the header (if they were not not given on the

original IMAGE keyword) or set automatically, assuming the image is part of

a contigous series, but the phi limits can be reset.

Then choose the "Find spots" option as described above. Note that there is a

limit to the total number of spots that can be stored internally, which may

place a limit on how many images can be used. Raising the spot finding

"Threshold" will reduce the number of spots found if this causes problems.

Note also that the REFIX autoindexing algorithm itself will use a maximum of

2000 spots.

3.1.3 Selecting images for autoindexing

If spots have been found on several images, then by default all of these

spots will be used for autoindexing. If, however, you only want to use the

spots from selected images, use the "Select images" menu option. The spots

found on each image are stored in a separate "slot", and the "slot" numbers

(rather than the image numbers) must be given when selecting the images (so

that images with the same image number can be used). If you wish to make a

fresh start, use the "Clear spots" menu option.

3.1.4 Running the autoindexing

Autoindexing uses either the FFT based algorithm from DPS (Stellar,

Bolotovsky and Rossmann, (1998) J. Appl. Cryst. 30, 1036-1040) or Wolfgang

Kabsch's REFIX program (Kabsch, 1988, 1993) both of which have been

incorporated into the MOSFLM program.

Autoindexing is performed by selecting the "Autoindexing" menu item. The

program will present a list of possible unit cells and space groups, sorted

on the PENALTY of each solution, and the user has to select the appropriate

choice. (When using REFIX, if a crystal symmetry AND unit cell have been

applied then only solutions for this symmetry will be listed).

In this list, the first number is the number for that solution, the second

number is a score for that solution (headed "PENALTY"). This is followed by

the lattice type, the cell parameters and a list of possible spacegroups.

Normally one would choose the solution with the highest possible symmetry,

but which still has a reasonably low "PENALTY" (The LOWER the PENALTY the

better).

example:

18 150  cI   103.13   103.36   103.01    62.8  62.8  62.9  I23,I213,I432,

I4132

17  64  tP    74.44    74.54    74.56    92.6  92.2  92.4  P4,P41,P42,P43,

P422,P4212,P4122,

P41212,P4222,

P42212,P4322,P43212

16  63  oP    74.44    74.54    74.56    92.6  92.2  92.4  P222,P2221,P21212,

P212121

15  63  tP    74.54    74.56    74.44    92.2  92.4  92.6  P4,P41,P42,P43,

P422,P4212,P4122,

P41212,P4222,

P42212,P4322,P43212

14  62  hR   107.32   103.13   131.17    92.0  89.8 121.4  R3,R32

13  41  oC   103.01   107.80    74.44    90.2  93.3  90.0  C222,C2221

12  41  mP    74.54    74.44    74.56    92.2  92.6  92.4  P2,P21

11  41  mP    74.56    74.44    74.54    92.4  92.6  92.2  P2,P21

10  40  oC   103.01   107.80    74.44    89.8  93.3  90.0  C222,C2221

9  40  mP    74.54    74.44    74.56    92.2  92.6  92.4  P2,P21

8  26  cP    74.54    74.56    74.44    92.2  92.4  92.6  P23,P213,P432,

P4232,P4332,P4132

7  24  mC   103.36   107.32    74.54    89.8  93.6  90.1  C2

6  22  mC   103.36   107.32    74.54    89.8  93.6  90.1  C2

5  19  aP    74.44    74.54    74.56    87.4  92.2  87.6  P1

4   6  hR   107.32   107.52   123.58    89.9  90.0 119.8  R3,R32

3   4  mC   103.36   107.32    74.54    90.2  93.6  89.9  C2

2   2  mC   103.01   107.80    74.44    89.8  93.3  90.0  C2

1   0  aP    74.44    74.54    74.56    92.6  92.2  92.4  P1

In this case R3 or R32 is an obvious choice.

Once you have made a selection the autoindexing is repeated automatically

imposing the appropriate cell constraints. You are then given the choice of

accepting that solution or trying another one from the list. REMEMBER that

the true spacegroup can only be determined from reflection intensities, NOT

from unit cell parameters.

BEWARE of monoclinic spacegroups with beta angles close to 90 being

misclassified as orthorhombic etc.

The final orientation (A matrix) and cell parameters are written to a file,

which can be defined when the autoindexing procedure is initiated, or with

the keyword NEWMAT (defaults to NEWMAT).

The file can be read in (MATRIX keyword) in future processing jobs.

If you want to change (permute) the order of the cell axes, simply include a

CELL keyword giving the cell that you would like to fit. For example, in

orthorhombic space groups the autoindexing will select the cell with a < b <

c. If the cell dimensions are 50, 100, 150 and you want to have a=150, b=50,

c=100 give the keyword:

cell 150 50 100

before running the autoindexing.

3.1.4.1 How do you know if it has worked correctly ?

The most obvious test of the success of the autoindexing is to predict the

pattern using the "Predict" menu option and see if it matches the observed

pattern.

If there was a large error in the input direct beam coordinates, with the

REFIX autoindexing this is sometimes apparent in a shift of the predicted

pattern relative to the observed spots. This shift can be corrected using

the "Adjust" menu option. With the DPS indexing, the direct beam cordinates

are automatically updated, so it should not be necessary to "Adjust" the

pattern. If the shift is significant, it is probably worth repeating the

autoindexing with the updated direct beam parameters (they are updated

automatically by using "Adjust") as this will give more accurate cell

parameters.

The single most important number by which to judge whether autoindexing has

succeeded is the positional residual (standard deviation of spot position).

This value should be below 0.2-0.3mm. If it is above 0.3mm the solution is

highly suspect, and if above 0.4mm it is almost certainly wrong. Values of

0.08mm to 0.12mm are typical for a correct solution (Note that the

positional residual will depend on the size of the diffraction spots. The

values given here are for a spot size of about 6x6 pixels with a pixel size

of 0.15mm, larger spots will give slightly larger residuals).

3.1.4.2 Errors reported by the autoindexing

Possible options if the autoindexing fails are:

1) Make sure the direct beam coordinates are correct ! (The autoindexing is

quite sensitive to these). If necessary, record a powder pattern (eg from

bee's wax or parrafin wax) and display this image and work out the

coordinates of the centre of the rings.

2) Try changing the intensity threshold "Min(I)/sig(I)" in the "Processing

Parameters menu" (up or down).

3) For the new DPS indexing, change the maximum cell edge.

3) Include data from other images (this could also give a more accurate

cell).

4) Try to avoid images looking down a principle zone.

5) If you know the cell and spacegroup, the original version of REFIX (which

requires input of a cell and the spacegroup) sometimes works when the new

one does not !! Use menu option "Save spots" to write out a spot list which

can be input to the original version of REFIX.

****BEWARE**** The program writes a list of spots when running the

autoindexing from within MOSFLM, but this file has a different format to the

one written using the "Save spots" menu option, and CANNOT be read by the

original REFIX so don't try it !

3.2 Autoindexing when running the program in background

(Note that as far as MOSFLM is concerned, a job that directs output to a

file rather than to a terminal is considered to be a background or batch

job).

It is only possible to autoindex in a background job if the spacegroup and

cell are known and this can only be done with the REFIX autoindexing.

Autoindexing is invoked by including the keyword AUTOINDEX. If no images are

specified, the first image to be integrated (specified on the PROCESS

keyword) will be used for autoindexing.

Thus :

CELL 107 107 123 90 90 120

SYMM R32

AUTOINDEX

PROCESS 1 TO 30 [ ANGLE 1.0 START 0.0 ]

will autoindex using image 1 and then integrate images 1 to 30.

Note that the cell derived from the autoindexing, rather than that given by

the CELL keyword, will be used during integration. If the cell is known

accurately it is usually better to override the cell derived from

autoindexing by using the KEEP keyword:

CELL KEEP 107.73 107.73 123.59 90 90 120

If you want to include more than one image in the autoindexing they can be

specified explicitly:

AUTOINDEX IMAGES 1 2 3

will use the first three images. In this case it is assumed that the phi

values are read from the image header, or that these images form part of a

contiguous rotation in phi.If this is not the case, the phi values can be

specified explicitly:

AUTOINDEX  IMAGE 1 PHI 0 1 IMAGE 20 PHI 50 51

If the image identifier (used to form the template for the image filename)

is not the same for all images, it can also be specified explicitly:

AUTOINDEX IMAGE 1 PHI 0 1 IMAGE 20 PHI 50 51 IDENT test_2

Note that if PHI or IDENT are given, then only ONE image can be specified on

each IMAGE keyword so that:

AUTOINDEX IMAGE 1 PHI 0 1 IMAGE 20 21 IDENT test_2  is NOT allowed

The "Min I/sig(I)" threshold can be set also:

AUTOINDEX THRESHOLD 30

Parameters asociated with spot finding can be set with the FINDSPOTS

keyword:

eg:

FINDSPOTS THRESHOLD 10 RMIN 25 RMAX 75 SPLIT 0.5 0.5

FINDSPOTS MINX 0.5 MAXX 1.5 MINY 0.5 MAXY 1.5 XOFFSET 25

----------------------------------------------------------------------------

Back to Contents Page

4: Running the STRATEGY option

4.1     Overview of the STRATEGY option

4.2     Some Examples of the STRATEGY options

4.3     Determining the oscillation angle for each

image (TESTGEN option)

----------------------------------------------------------------------------

Having determined the crystal orientation, one then needs to know what

rotation range is necessary to collect a complete (or essentially complete)

dataset. The STRATGEY option provides a very rapid and convenient means for

doing this.

4.1 Overview of the STRATEGY option

The STRATEGY option allows the design of a data collection strategy in a

semi-automatic way for a single axis rotation camera. It requires all the

parameters normally used to process a set of images (crystal symmetry,

orientation, crystal to detector distance, wavelength, detector type, direct

beam position).

The rotation range (PHITOT) required to collect a complete dataset is

determined from the crystal symmetry and orientation (eg 180 degrees for

Laue group P2/m if rotating about the b axis, 90 degrees if rotating about a

or c).

The phi value (PHIZONE) which, for orthorhombic (or lower) symmetry places

an axis in the XZ plane (containing the X-ray beam and the rotation axis),

or for trigonal or higher symmetries places the unique symmetry axis in the

plane normal to the X-ray beam and containing the rotation axis, is

determined. A reflection list corresponding to a total rotation of PHITOT

starting at phi=PHIZONE is generated.

For orthorhombic spacegroups the algorithm used to calculate PHIZONE is not

foolproof ! It works in approximately 90-95% of cases. When it does not

work, the predicted completeness may be up to 3-4% less than what could be

achieved using a different value of PHIZONE. If the predicted completeness

is less than expected, try giving the ALTERNATE keyword as part of the

STRATEGY command. This will use a different value for PHIZONE which may

(rarely) give a higher completeness. As a rule of thumb, it should be

possible to get at leat 90% completeness for a total rotation of 60 degrees

in two segments.

To save time, the true unit cell will be "shrunk" when generating the

reflection lists. This can be controlled by the SPEEDUP subkeyword, but the

program will calculate a sensible default SPEEDUP if none is specified.

This reflection list is then compared to a list of all unique reflections

for this spacegroup and the completeness and multiplicity is calculated,

both as a function of rotation and resolution.

It is assumed that all possible reflections are measured (ie none are lost

because of spatial overlaps or because they extend over too many images).

However, some reflections may be unobserved because they lie in the cusp

region. The percentage of reflections within the cusp will depend on the

wavelength, crystal symmetry and crystal orientation, and can be minimised

by trying to orient the crystal so that the crystal axis closest to the

rotation axis is at least THETAMAX degrees AWAY from the rotation axis,

where THETAMAX is the maximum Bragg angle.

It is often possible to collect data with a very high percentage

completeness with a total rotation significantly less than PHITOT. This will

inevitably result in a lowering of the overall multiplicity, but if data

collection time is limited (for example at a synchrotron source) it is

preferable to obtain a dataset with high completeness and less than optimal

multiplicity rather than an incompete dataset with higher multiplicity !

Equally, if radiation damage is a serious problem, it is best to get a

complete dataset first, and then collect additional images to increase the

multiplicity.

If the total rotation angle to be collected is specified, and the number (up

to 4) of discontinuous segments to be used, the program will determine the

start and end phi values for each segment that will give the highest

possible completeness. For example, a total rotation of 60 degrees in 2

segments for an orthorhombic spacegroup will result in the identification of

two 30 degree segments which give the highest completeness.

If some data has already been collected from one (or more) previous

crystals, the program will determine the starting phi value for the

"current" crystal that will give the maximum completeness, with the

assumption that the phi rotation for this crystal is such that the TOTAL

rotation for ALL the crystals is PHITOT (This assumes that all crystals are

mounted about the same axis). The user may also define the total rotation

angle for the current crystal.

4.2 Some Examples of the STRATEGY option

Once an image has been autoindexed, select the "Strategy" option from the

menu. The input for the STRATEGY option has to be given in the I/O window,

initially at the MOSFLM => prompt.

4.2.1 No previous data has been collected

Enter the following keywords:

STRATEGY

GO

The program will determine the phi angle PHIZONE (see above), and generate a

reflection list starting at that phi angle, for a total rotation determined

by the Laue group. It will then generate a list of all unique reflections

and merge the two lists. Finally it will give the completeness of the data

for the rotation range generated:

Optimum rotation gives  98.0% of unique data

This corresponds to the following rotation ranges for the final run

From   20.0 to 110.0 degrees

Type "STATS" for full statistics

....

STRATEGY =>

Typing STATS at the prompt will give a breakdown as a function of rotation

angle and resolution, and a breakdown of the anomalous data.

If insufficient time is available to collect the full rotation range

required, one can determine the best segments to collect to achieve maximum

completeness. Type at the STRATEGY => prompt:

STRATEGY => ROTATE 50 SEGMENTS 2

STRATEGY => GO

(*** The ROTATE keyword MUST be given before the SEGMENTS keyword **) The

program will then give the best phi ranges to collect for two segments, each

of 25 degrees, giving a total rotation of 50 degrees:

Optimum rotation gives  96.1% of unique data

This corresponds to the following rotation ranges for the final run

From   20.0 to  45.0 degrees

From   65.0 to  90.0 degrees

One can try using 3 segments of data instead of two:

STRATEGY => rotate 50 segments 3

STRATEGY => go

In this case, the result is:

Optimum rotation gives  98.0% of unique data

This corresponds to the following rotation ranges for the final run

From   20.0 to  37.0 degrees

From   52.0 to  69.0 degrees

From   74.0 to  90.0 degrees

The effect of using other total rotations and different numbers of segments

can also be tested (but using more than 3 segments is very time consuming

and in fact there is an absolute limit of 4 segments).

Alternatively, the completeness of specified segments can be tested:

START 0 END 20

START 65 END 90

GO

Note that the phi ranges specified on the START and END keywords MUST lie

within the phi range generated by the program when it first starts. Thus if,

for example, the program has generated reflections from phi=10 to phi=100

then it not possible to try:

START 0 END 30

GO

or ROTATE 100

at the STRATEGY prompt (The program will complain).

4.2.2 When some data have already been collected

It is also possible to deal with the case where some data have already been

collected (from the same or from other crystals).

a) Data from the same crystal

Consider the case where 30 degrees of data have been collected (from phi =

-10 to phi = 20 say), and we want to determine how best to complete the

dataset with an additional rotation of 40 degrees.

Select the "Strategy" menu option, and enter the following keywords:

STRATEGY START -10 END 20 PARTS 2 SPEEDUP 10 (Note use of SPEEDUP keyword)

GO

STRATEGY ROTATE 40 SEGMENTS 2

GO

The program will then find the phi values for the two segments (each of 20

degrees) which when combined with the 30 degrees of data already obtained

will give the maximum completeness.

b) Data from different crystals

Imagine that data have been collected from phi = -20 to 15 from an

orthorhombic crystal with an orientation matrix "xtal_1.mat"

A second crystal is mounted, an image collected and it is autoindexed to

give an orientation matrix "xtal_2.mat". The STRATEGY option can now

determine the best phi range for this second crystal to complete the data.

First, specify the orientation of the first crystal using "Keyword Input"

and the keyword MATRIX:

MATRIX xtal_1.mat

Then autoindex the first image of the second crystal. This MUST be done

AFTER the orientation matrix for the first crystal has been specified,

because the orientation of the second crystal has to be referred to that of

the first.

The select the "Strategy" menu option, and enter the following keywords:

MATRIX xtal_1.mat

STRATEGY  start -20 end 19 PARTS 2 SPEEDUP 10

GO

MATRIX xtal_2.mat

STRATEGY AUTO

GO

Normally the first crystal will have been collected starting at a zone. If

this is NOT the case, it will probably be necessary to collect two segments

of data from the second crystal to get complete data. This can be done by

specifying "STRATEGY AUTO SEGMENTS 2" for the second crystal, and it may be

advantageous to specify the sizes of the two segments. Thus if the first

crystal was collected starting at 15 degrees away from a zone, for a total

of 35 degrees, then the second crystal will need one segment of 15 degrees

and another of 40 degrees (90-35-15) go get best completeness.

It is also possible to automatically find the best rotations(s) for a

smaller total rotation. Once the program has come up with the STRATEGY =>

prompt (ie after it has found the best solution for a single 55 degree

rotation in the above case) one can then type:

STRATEGY => PART 1                    ! Include all data from first crystal

STRATEGY => AUTO ROTATE 40 SEGMENTS 2 ! Use 2 segments (each 20 degrees for

! second crystal

This means include ALL data that has already been collected (from -20 to +19

in the above example) and then determine the best phi values giving a total

rotation of 40 degrees (in two 20 degree segments) from the second crystal.

4.2.3 Optimising anomalous data collection

To optimise the number of anomalous pairs rather than the completeness of

the unique data simply include the subkeyword ANOMALOUS:

STRATEGY ROTATE 60 SEGMENTS 2 ANOMALOUS SPEEDUP 10

This will not necessarily be the same phi range(s) as that which maximise

the overall completeness.

4.2.4 A complete list of the STRATEGY subkeywords

STRATEGY subkeywords

subkeywords: AUTO ROTATE SEGMENTS SIZES START END PARTS SPEEDUP ANOMALOUS

AUTO Determine the starting phi angle and the phi rotation required to give

a complete dataset (if possible from a single crystal setting), and give

statistics on completeness and multiplicity. Do NOT use START or END with

the AUTO keyword. This is the default mode of running stratgey.

ROTATE <phirot> Only for use with the AUTO option. Restrict the total

rotation to "phirot" degrees.

SEGMENTS <nseg> Only for use with the AUTO option. Allow "nseg" discontinous

segments of data to give a total rotation of PHIROT degrees. Unless

specified explicitly with the SIZES keyword (see below) the segments will

have approximately equal widths in phi.

SIZES <size1,size2,size3...> The sizes for the "nseg" segments. If SIZES are

given, then the "phirot" value given on the ROTATE keyword is ignored, and

the total rotation is the sum of the SIZES. Default: Use approximately equal

sizes with total "phirot"

START <phistart> END <phiend> As an alternative to AUTO mode, specify the

start and end phi values to be used in generating the reflection list. Up to

10 different sets of START and END can be given on successive STRATEGY

keywords. eg STRATEGY START 0 END 30 STRATEGY START 35 END 60 STRATEGY START

70 END 90

PARTS <nparts> If some data have already been collected (from the same or

other crystals), set "nparts" to the total number of segments of data

already collected plus one (which is the current crystal or segment whose

phi range is to be determined). This need only be given on the first

STRATGEY keyword.

SPEEDUP <n> Speed up the calculation by a factor "n".

ANOMALOUS Optimise anomalous pairs rather than completeness of data.

4.3 Determining the oscillation angle for each image (TESTGEN option)

The completeness analysis assumes that NO reflections are spatially

overlapped. Providing that spots within a lune (ie in the same plane in

reciprocal space) are not overlapping, spatial overlaps can usually be

reduced to an acceptable level by an appropriate choice of oscillation angle

for each image.

The TESTGEN option will calculate the maximum allowed oscillation angles as

a function of the phi value for a given maximum acceptable percentage of

overlapped reflections (which can be zero).

The determination of whether or not a reflection is spatially overlapped

depends crucially on the mosaic spread, beam divergence parameters and the

minimum allowed spot separation. The mosaic spread and the minimum spot

separation can be reset at the STRATEGY prompt to test how critical these

values are, using keywords MOSAIC and SEPARATION respectively.

Remember that for post-refinement to work, the oscillation angle must be

more than half the sum of the mosaic spread and beam divergence.

Note that the TESTGEN keyword can be given at the STRATEGY prompt or at the

normal MOSFLM prompt (without running the STRATEGY option).

Keywords:

TESTGEN subkeywords

subkeywords: START END STEP OVERLAP MINOSC MAXOSC ANGLE

START <phstart> Define the starting phi. This keyword MUST be given.

END <phend> Define the ending phi. This keyword MUST be given.

STEP <phstep> The optimum rotation angle will be calculated every "phstep"

degrees between "phstart" and "phend". Default: 5 degrees

OVERLAP <x> The maximum rotation angle giving less than x% overlapped

reflections will be calculated. Note that x is in PERCENT. Default 0%

MINOSC <rotmin> MAXOSC <rotmax> Only rotation angles between "rotmin" and

"rotmax" will be considered. Default: rotmin 0.2, rotmax 5.0

ANGLE <oscang> If the ANGLE keyword is given, then the overlap for a fixed

oscillation angle "oscang" is calculated between phi=phstart and phi=phend.

No attempt to find the "best" oscillation angle is made.

example:

TESTGEN START 0 END 90 OVERLAP 3 MINOSC 0.5

Exiting the STRATEGY option

Use keyword EXIT to end the strategy option.

An Example command file when not using the X-window menu

In this case, the type of detector (MAR,SMALLMAR,RAXIS etc) has to be

specified, and the crystal to detector distance and wavelength as these

cannot be read from the image header.

STRATEGY AUTO

DISTANCE 80

DETECTOR SMALLMAR

MATRIX lyso_1.mat

SYMM 19

BEAM  90 90

DIVERGENCE 0.35 0.3

MOSAIC 0.2

SEPARATION 1.5 1.5

POLARISATION MONOCHROMATOR

WAVELENGTH  1.5418

RUN

----------------------------------------------------------------------------

Back to Contents Page

5: Determining Accurate Cell parameters

5.1     Using Post-refinement to refine the cell

5.1.1   Doing post-refinement interactively

5.1.2   Doing post refinement in background

5.1.3   What the program actually does

5.1.4   Using several segments or different crystals

5.1.5   Tips on post-refinement

5.1.5.1 Processing images showing strong diffuse scatter

----------------------------------------------------------------------------

The unit cell parameters are refined as part of the autoindexing, but in

general not all the parameters will be well defined (in particular, the cell

parameter along the X-ray beam direction is ill-determined). Improved values

can be obtained by using two or more images widely separated in phi for the

autoindexing. However, accurate cell parameters are best determined by

post-refinement, for which it is necessary to have a number (at least two)

of abutting oscillation images. To obtain accurate cell parameters for

orthorhombic or lower symmetry spacegroups, it is essential to have data

from two orientations widely separated in phi, but for trigonal or higher

symmetry only one "block" of data is normally required.

A pragmatic procedure is as follows:

If more than about 15 degrees of data are available from a single crystal,

or several crystals in approximately the same orientation (within 20

degrees) use the "Refine cell" menu option (or the POSTREF SEGMENT option if

running in background) to get an accurate cell and then do NOT refine it

during integration.

If less than 15 degrees is available, use the refined cell from the

autoindexing in processing and try post-refinement using an angular wedge of

data, but if this is unstable (large sd's or shifts from cycle to cycle)

then fix the cell parameters, as the values from the autoindexing, while

they may be in error, will be sufficiently accurate to process a "local

region" in reciprocal space, ie up to 10-15 degrees from the starting phi

value.

5.1 Using Post-refinement to refine the cell

Post-refinement uses the distribution of the intensity of partially recorded

reflections over the two images on which the partial is recorded to refine

cell parameters, orientation and mosaic spread. It has the distinct

advantage that the derived cell parameters are entirely independent of all

detector parameters (crystal to detector distance and detector orientation)

and distortions (ROFF and TOFF) which, if inaccurate, can lead to

significant errors in the cell parameters derived from autoindexing.

**** IMPORTANT ****

Post-refinement can ONLY use partially recorded reflections which extend

over two images. Those which extend over three or more images CANNOT be

used. Thus if the mosaic spread (plus beam divergence) is more than twice

the oscillation angle, post-refinement will NOT be possible. Under these

circumstances, a larger oscillation angle should be used, or alternatively

the data can be processed in "blocks" of images, determining a new

orientation for each block by autoindexing.

5.1.1 Doing post-refinement interactively

Having obtained the crystal orientation by autoindexing, choose the "Refine

cell" menu option. You can then select the number of "segments" of data to

use in the refinement, the first image and the number of images to be used

in each segment. Note that there must be at least two images in each

segment, but there is generally little to be gained from using a total of

more than 5-10 images in ALL segments (unless there are only a few partials

on each image).

Note that when using data from two segments widely separated in phi, it is

possible that the crystal orientation will have changed sufficiently that

the orientation matrix for the first segment of data does not accurately

predict the first image of the second segment. This can be quickly checked

by reading in this image ("Read image" menu option) and then predicting the

pattern ("Predict"). If the prediction is poor, there are two things that

can be done. Either find spots on this second image and use them (together

with the spots from the first image of the first segment) to repeat the

autoindexing. This may give a matrix that predicts both images successfully.

This should work unless the crystal orientation has genuinely changed

between the two images (or the rotation axis is not normal to the X-ray

beam). If this doe not work, you should derive a new orientation matrix for

image from the second segment image. Remember to change the name of the file

that the orientation matrix will be written to. REMEMBER to delete all the

spots used to autoindex the first image if you have not already done so.

Then use the "Autoindex" option to get an orientation matrix for this image.

Under these circumstances, it is best to FIX the cell parameters for the

autoindexing to those determined for the first segment of data (not possibel

for DPS indexing). This is because only a single set of cell parameters is

allowed (for all segments) when doing the post-refinement. The "Refine cell"

procedure allows you do define a separate orientation matrix for each

segment of images.

Because the post-refinement uses partially recorded reflections, it is

important to have a realistic estimate of the mosaic spread BEFORE starting

post-refinement. In particular, if no value has been supplied (ie the mosaic

spread is zero) the program will issue a warning message because it is

unlikely that the post-refinement will work. The simplest way to obtain an

initial estimate of the mosaic spread is to "Predict" the pattern for

several values (eg 0.1,0.2,0.3,0.5,0.75,1.0 etc) and see which value gives

the best fit to the observed pattern. The postrefinement will give a refined

estimate of the mosaic spread.

5.1.2 Doing post refinement in background

To use this option, the keyword :

POSTREFINEMENT SEGMENT <number of segments>

should be used, followed by PROCESS keywords defining the images to be

included in each segment, with each PROCESS (see 5.3.1) keyword followed by

a RUN keyword.

Example:

NEWMAT postref_3seg.mat      ! Defines the filename for the new matrix

POSTREF SEGMENT 3

PROCESS 1 3 [ ANGLE 1.0 START 0.0 ]

RUN

PROCESS 43 45 [ ANGLE 1.0 START 42.0 ]

RUN MATRIX test_88.mat

PROCESS 86 88 [ ANGLE 1.0 START 85.0 ]

RUN

Would use 3 segments of data (with phi values 0-3,42-45,85-88.) Note that a

new MATRIX keyword has been given for the last segment, which could be

necessary if the crystal has slipped during data collection. See section

5.1.1 for the best procedure to use when deriving an orientation matrix for

the second or subsequent segments.

Note that the procedure uses only partially recorded reflections, and so in

this case it would use partials then span images 1 and 2, 2 and 3, for the

first segment etc. For this reason the PROCESS keyword MUST specify at LEAST

2 images.

PROCESS 1 1 ANGLE 1.0 START 0.0 would provide NO data for post-refinement.

5.1.3 What the program actually does

During postrefinement, the images are not fully integrated (only the

intensities of partially recorded reflections are measured, and by summation

integration rather than profile fitting) so there is no output generate file

or MTZ file. The crystal orientation will be refined for every image

independently, but the cell parameters will only be refined once the final

segment of data has been processed

(Note that the very last image (88 in the example above) is apparently (from

the logfile) not measured at all...this is NOT an error, since the

intensities of the partials at the start of image 88 are obtained while

processing image 87.)

If the cell parameters change by more than 2.5 standard deviations from the

input values, all images will be remeasured using the updated cell and

another round of cell parameter post-refinement will be carried out. This

will happen up to a maximum of 4 repeats. It is quite common that two or

even three complete rounds of integration are required for convergence. For

this reason it is not a good idea to include too many images in the

refinement. A target of between 500 and 2000 reflection in the refinement is

perfectly adequate.

It is recommended that the final cell parameters are then used to integrate

all the images in the dataset, fixing the cell parameters in the

post-refinement:

POSTREF FIX ALL

5.1.4 Using several segments or different crystals

Note that if the crystal has been slipping during data collection, it is

possible to provide different MATRIX keywords for each segment of data, and

supply a new orientation (eg derived by autoindexing the first image of the

segment). When doing this, the orientation matrices for all segments

(including the first) SHOULD BE OBTAINED FROM THE SAME INTERACTIVE RUN OF

MOSFLM. This ensures that the matrices for the second and subsequent

segments are all referred relative to the orientation matrix for the first

segment. It is also a good idea to FIX the cell parameters when autoindexing

the images from the second and subsequent segments, as only one set of cell

parameters is allowed when refining the cell by post-refinement. It is also

possible to provide new crystal identifiers for each segment (eg if the

crystal has been translated and the images given a different identifier). It

is also possible to use data from different crystals, but in this case there

is the restriction that the orientation of the crystals must be the same (to

within 20 degrees) and the relative phi values must be correct. Providing

the different crystals are all indexed in the same run of MOSFLM, the

relative phi values are taken care of automatically.

A possible complete example is then:

TITLE  Refine cell with 3 segments

DIVERGENCE 0.35 0.2

SYMMETRY 96

[DISTANCE 124.1]

[WAVELENGTH 1.542}

DIRECTORY /scr0/andrew/

BEAM 89.33 90.10

GAIN 1.2

NEWMAT postref_3seg.mat

POSTREF SEGMENT 3

IDENT oval1

MATRIX oval1.mat

PROCESS 1 3 [ANGLE 1.0 start 0.0]

RUN

IDENT oval2

MATRIX oval43.mat

PROCESS 43 45 [ANGLE 1.0 START 42.0]

RUN

IDENT oval3

MATRIX oval86.mat

PROCESS 86 88 [ANGLE 1.0 START 85.0]

RUN

When doing post-refinement, the crystal orientation around the X-ray beam

direction (the X axis) is not defined (the refinement is based solely on the

observed degree of partiality and not on the positions of the spots) and

this parameter is therefore not refined, but missetting angles around Y and

Z axes are refined (see Appendix IV for a definition of coordinate frames).

The refinement of the detector parameter CCOMEGA allows for crystal slippage

around the X-ray beam direction.

If only a narrow angular wedge of data is available for a low symmetry

spacegroup (orthorhombic or lower) it is possible to FIX cell parameters

that are not well defined (those closest to the direction of the X-ray beam)

eg POSTREF FIX A

5.1.5 Tips on post-refinement

In the great majority of cases the post-refinement will provide accurate

cell parameters without any user intervention (providing the mosaic spread

estimate is realistic). There are, however, some special cases where

additional input is required to get the best results.

5.1.5.1 Processing images showing strong diffuse scatter.

It is not uncommon to observe diffuse scatter on the images, particularly

for data collected at a synchrotron source. Sometimes this takes the

appearance of a "halo" around the Bragg spot, because the intensity of some

types of diffuse scatter peak at the positions of the Bragg reflections.

This can cause difficulties in post-refinement, because it has the same

effect as a crystal with a very large mosaic spread. Under these

circumstances, it is best to refine the cell parameters using spots that are

close to half-recorded, as the refinement is then less sensitive to the

model for the "rocking curve". The minimum and maximum fraction recorded can

be specified as shown below:

POSTREF FRMIN 0.4 FRMAX 0.6

will only use reflections that are between 0.4 and 0.6 recorded. (Default is

0.1 to 0.9).

----------------------------------------------------------------------------

Back to Contents Page

6: Collecting data and processing the images

6.1     Overview

6.2     Special MOSFLM features

6.2.1   Accumulating profiles over several images

6.2.2   Addition of partials (ADDPART)

6.2.3   Post-refinement of orientation and cell

parameters

6.2.4   Optimisation of measurement box parameters

6.3     Running a processing job

6.3.1   Running MOSFLM interactively

6.3.2   Processing the first block of data)

(Non-interactively)

6.3.3   Finally, Processing the dataset

----------------------------------------------------------------------------

6.1 Overview

Before starting the serious data collection, integration of one or more

images should be carried out to determine:

a) Is the crystal single?

b) Is the exposure time correct?

c) Is the crystal to detector distance correct (ie the whole of the detector

is being used)?

d) Can the images be processed...are the spots separated and is the number

of spatial overlaps small?

6.2 Special MOSFLM features

There are 4 features of MOSFLM which are unusual and require explanation.

These are:

1. Accumulation of standard profiles over several images.

2. Addition of partially recorded reflections over adjacent images.

3. Post-refinement of cell parameters and crystal orientation.

4. Optimisation of the measurement box parameters.

6.2.1 Accumulation of profiles

In order to form well defined standard profiles (which are then used to

evaluate the profile fitted intensities) fully recorded (or partially

recorded) reflections over several images are added together. This improves

the signal to noise and results in a better determined profile. The number

of images used to form the profiles (usually between 5 and 10) is determined

automatically by the program (in a way that avoids having just a few images

in the final block). It can also be set manually by the BLOCK subkeyword on

the PROCESS keyword line.

The positional refinement for all images in a block is carried out prior to

forming the standard profiles and integrating the images. Thus each image is

processed in two passes, the first pass for the positional refinement and

writing all the "measurement boxes" for the spots to the SPOTOD file, and

the second for actually evaluating the reflection intensities.

6.2.2 Addition of partials (ADDPART option)

The program has the option to add together the measurement boxes of the two

halves of partially recorded reflections on adjacent images, thus giving the

equivalent fully recorded reflection which can then be used to form standard

profiles or for positional refinement of the detector parameters.

To make use of this option, the keyword:

ADDPARTials

should be given. (The default is now NOT to add partials).

Note that this procedure, which involves adding pixel values on two adjacent

images, involves two assumptions:

Assumption 1

That the images have the same effective exposure time (ie total incident

flux). If the rate of rotation of the spindle axis is determined by the

ionisation chamber reading (as it may be on the MAR detector) then this

assumption should be met. If not, then there may be an error introduced by

this procedure especially if the incident beam is rapidly decaying (eg on an

unstable synchrotron source). If post-refinement is being used (and by

default it is used) then the program will print a warning message (to the

summary file and the end of the logfile) if the exposure varies by more than

5% from one image to the next (as judged by the X-ray background).

Assumption 2

That the detector origin, orientation etc is identical for successive

images, and that the images are exactly abutting (ie no overlap in rotation

angle). These conditions will normally be met by the Mar, R-axis and Mac

Science scanners, but mechanical wear can lead to the scanner not locking

into the correct "home" position after a scan (it does one more or one too

few rotations). This will show up as a variation in the ROFF distortion

parameters in units of one pixel (0.15mm). The program keeps track of

variations in ROFF,TOFF and CCOMEGA and will give a warning message if undue

variation is detected.

**** IMPORTANT *****

If either of these assumptions is not met (this will be indicated by warning

messages) then the ADDPART option should not be used.

With ADDPART, what are actually partially recorded reflections over 2 images

are reclassified as fully recorded when stored in the MTZ file and they will

therefore be used in scaling (SCALA or ROTAVATA). However, summed partials

do carry a special flag, so that they are still classified as partials in

the statistical analysis in SCALA or AGROVATA. Thus information on partial

bias, for example, is still available.

Because of the ability of SCALA to scale data when there are no fully

recorded reflections, the use of this option less important than it once

was. Because its use depends on the assumptions listed above, which may not

always be met, the DEFAULT is now NOT to add partials.

6.2.3 Post-refinement of cell parameters and crystal orientation

By default the program will refine both cell parameters and crystal

orientation using post-refinement during integration of the images. However,

it is in fact preferable to determine accurate cell parameters prior to

integration using the Refine cell menu option for interactive work or the

POSTREF SEGMENT option in a background job. The resulting cell parameters

are then input using a CELL keyword and the cell is NOT refined during

integration (by using keywords POSTREF FIX ALL). This will refine the

crystal orientation (and mosaic spread) but not cell parameters.

If cell parameters are refined in a processing job, the way in which the

refinement is carried out depends on the crystal spacegroup. For crystals of

trigonal or higher symmetry data from each pair of images in turn is used in

the refinement. (This is equivalent to the POSTREF SINGLE mode.) Thus cell

parameters, crystal orientation and mosaic spread are refined after every

image using intensities on that image and the next one in the series. (For

off-line scanners, reflections on the current image and the preceeding image

are used).

For lower symmetry this is not recommended, because not all the cell

parameters will be well defined using data from only one pair of images.

Thus for orthorhombic and lower symmetries data is accumulated from a number

of images and only then will cell parameter refinement be carried out (the

crystal orientation is still refined after every image as this is well

defined). By default, the number of images required for the cell parameter

refinement (NADD) is set to correspond to a rotation of 10 degrees. However,

this can be changed using the WIDTH subkeyword. Thus:

POSTREF WIDTH 15

specifies that 15 degrees of data must be accumulated for post-refinement of

cell parameters. The actual WIDTH of data required for a satisfactory

refinement will depend on the resolution (the higher the resolution, the

fewer images are required) and the strength of the data (the weaker the

data, the more images are required). Some experimentation may be required to

find a WIDTH that gives a stable refinement. If the refinement appears

unstable (ie large shifts in cell parameters) the WIDTH should be increased.

If this is not possible (eg only a limited number of images have been

obtained from the crystal before radiation damage set in) then the

refinement of some or all cell parameters should be turned off. Thus

POSTREF FIX ALL

will fix all cell parameters.

POSTREF FIX C

will fix the "c" cell parameter etc. Normally one would fix the cell

parameter that is closest to being parallel to the X-ray beam as this will

be the least well defined. Alternatively, look at the standard deviations of

the cell parameters to see which one(s) are least well defined. Normally the

cell parameters obtained from autoindexing are quite adequate to measure

10-20 degrees of data from the image on which the autoindexing was run.

Once the appropriate number of images (NADD) have been processed, and the

cell parameters have been refined for the first time, if there is a large

shift in any cell parameter the program will start processing from the first

image again, using the updated cell parameters. (The maximum shift allowed

is determined by the subkeyword SHIFTFAC..thus

POSTREF SHIFTFAC 5

sets the maximum shift to 5 standard deviations...if larger than this the

images will be reprocessed. The default value is 2.5.

From this point on, the cell parameters will be re-refined after every

image, using data from the previous NADD images. For example, with 1 degree

oscillation images and a width of 10 degrees, the first cell refinement will

be carried out after processing image 10, using data from images 1 to 10.

After processing image 11, cell parameters will be refined using data from

images 2-11 etc. etc.

The missetting angles should ALWAYS be refined by post-refinement, but it

may be necessary in some cases to suppress or limit refinement of cell

parameters if the refinement is not stable.

The crystal mosaic spread is also refined by default, but the refined value

IS NOT USED BY DEFAULT. This is because if the refinement is unstable, this

can have rather drastic effects on the processing. If the refinement is

stable, and there is evidence for a change in mosaic spread during the run

(this often results from radiation damage), the refined values should be

used by including the subkeyword USEBEAM:

POSTREF USEBEAM

If you wish to refine the horizontal and vertical beam divergence

independently (good data is required to do this) use BEAM 2 :

POSTREF BEAM 2

Again, you need to include USEBEAM to actually make use of the refined

values.

6.2.4 Optimisation of measurement box parameters

By default the program will automatically determine the best measurement box

parameters. It will first determine the spot size from spots in the centre

of the image (parameters for this search are set by keyword SPOT). This

information is used to set initial sizes for the overall dimesnions (NXS,NYS

in figure below) and the corner and rims parameters (NC,NRX,NRY). Following

detector parameter refinement using spots from the centre of the first

image, the program will then optimise the rim and corner parameters NRX,NRY

and NC.

<------------------ NXS = 23 --------------->

^    - - - - - - - - - - - - - - - - - - - - - - -  ^

!    - - - - - - - - - - - - - - - - - - - - - - -    NRY =2

!    - - - - - -                       - - - - - -  ^

!    - - - - -                           - - - - -

!    - - - -                               - - - -

!    - - -                                   - - -

!    - - -                                   - - -

!    - - -                                   - - -

NYS =17 - - -                                   - - -

!    - - -                                   - - -  ^

!    - - -                                   - - -  !

!    - - -                                   - - -  !

!    - - - -                               - - - -  !

!    - - - - -                           - - - - -  NC =8

!    - - - - - -                       - - - - - -  !

!    - - - - - - - - - - - - - - - - - - - - - - -  !

^    - - - - - - - - - - - - - - - - - - - - - - -  ^

<NRX> = 3

Figure 1. The measurement box used in MOSFLM. NXS and NYS (odd integers)

define the overall size of the measurement box in pixels. NRX and NRY define

the widths of the background rim and NC defines the corner cutoff. In the

figure a "-" denotes a background pixel, all other pixels belong to the

peak. There is no "safety rim" between peak and background.

The algorithm employed is as follows. The parameters NRX, NRY, NC are varied

in turn and the value giving the highest ratio of the integrated intensity I

to the standard deviation in the intensity sigma(I) is found. This is the

notional optimum value for that parameter. The total intensity for this

value is checked, and if it is less than the maximum intensity (for any

value of the parameter) by more than a factor 0.01 (TOLERANCE), then that

parameter is decreased by up to IBOUND pixels.

For example, the max I/sigma(i) might be found for a X rim value of 4, but

the intensity might be only 97% of the maximum intensity found for any value

of NRX (eg 1). NRX will therefore be decreased, one pixel at a time, and for

each value the integrated intensity tested against the maximum value. If for

NRX=2, the intensity is within 0.01 (ie 1%) of the maximum value, then 2

will be taken as the optimal value for NRX.

Thus it can be seen that the higher the TOLERANCE parameter, the SMALLER the

optimised peak area will be. It is difficult to define a "correct" value for

TOLERANCE, because this will depend on the degree of diffuse scatter

associated with the Bragg peaks and how well adjacent spots are resolved.

Values between 0.01 and 0.04 are typical, it should NOT be necessary to use

values above 0.04. Note that two values may be supplied to the TOLERANCE

keyword. In this case the first value is used for profiles in the centre of

the image (closest to the direct beam) and the second value for the

outermost profiles. An interpolated value is used for other profiles. The

default value for the innermost profile is 0.01. For very close spots this

can be increased to 0.02 (rarely larger).

For the first round of this iteration, when optimising NRX and NRY, NC is

set to zero. NRX and NRY are varied from 1 to a maximum value which would

give a peak dimension of 5 pixels. NC is varied from the smaller of NRX and

NRY up to the smaller of (NX-2) and (NY-2).

Two rounds of optimisation are performed, the second round using the results

of the first.

The optimisation is first carried out on the average spot profile for the

central region of the detector. The optimisation of the overall dimensions

(NXS,NYS) is only carried out at this stage. The background rim parameters

are optimised for ALL the standard profiles.

The background rim parameters are reoptimised for each new BLOCK of images.

If the optimisation of the standard profiles causes problems (because of

unusual spot shapes with long tails or other features) it can be suppressed

using keywords:

PROFILE NOOPTIMISE

In this case the measurement box parameters will still be optimised for the

average spot profile using spots from the centre of the detector (this

profile is NOT used for integration). This box will then be expanded

automatically to allow for the increase in spot size due to obliquity of

incidence on the detector, but the measurement box parameters will NOT be

optimised for the standard profiles (one for each area of the detector) that

are used for integration. To suppress the optimisation of the measurement

box parameters altogether (so that the program will use the parameters

supplied on the RASTER keyword), give the keywords:

PROFILE NOOPTIMISE ATALL FIXBOX

To just suppress the optimisation of the overall size of the measurement box

(parameters NXS,NYS) include keywords:

PROFILE FIXBOX

6.3 Running a processing job

Normally at this stage one would go straight on to process the first BLOCK

of images (See "6.3.2 Processing the first block of data" below).

6.3.1 Running MOSFLM interactively

The following keywords might be used for an interactive run. Before starting

MOSFLM, it is convenient to edit them into a file (eg comm) to save typing

them several times. Then at the mosflm prompt simply type:

@comm

and it will read the commands from the file

TITLE Processing test data

! Crystal parameters

MATRIX oval_2_3.mat

SYMMETRY 96

MOSAIC 0.1

! image parameters

IDENT oval

PROCESS 1 TO 1 [ ANGLE 1.0 START 0.0 ]

DIRECTORY /scr0/andrew/

EXTENSION image

! beam parameters

DIVERGENCE 0.35 0.2

POLARISATION MIRRORS

! detector parameters

BEAM 89.33 90.10

BACKSTOP CENTRE 88.5 91 RADIUS 14

GAIN 1.2

! The following are read from the image header if not supplied on

! keywords for Mar, ADSC, RaxisIV and Mac Science. The phi values (on

! PROCESS keyword) will also be read from the header if not supplied.

DISTANCE 124.1

WAVELENGTH 1.542

The following are optional, if not given, the program will set suitable

defaults.

!HKLOUT oval.mtz

!SEPARATION 1.3 1.3

!RASTER 17 17 9 3 3

!GENFILE oval_1to1.gen

!RESOLUTION 3.0

PLOT

RUN

*** NOTE WELL ***

If the data has been collected at a synchrotron source, the polarisation of

the beam and the horizontal and vertical divergences (horizontal here means

in the plane of the X-ray beam and the rotation axis) should be given.

Values default to those for the SRS at Daresbury, UK.

The TITLE is written to the mtz and generate files.

MATRIX is the filename for the orientation matrix derived from a previous

autoindexing or cell parameter post-refinement run. If you want to override

the cell parameters or missetting angles in this matrix use the CELL and

MISSETTS keywords respectively.

SYMMETRY gives the space group of the crystal, either as the name or the

number in International Tables. Note that axial systematic absences (eg 0k0

with k odd in spacegroup P21) are measured by MOSFLM, so that the symmetry

can be checked. Lattice absences however (due to face or body centring) will

NOT be measured.

The IDENTifier (oval) is used as a template for the image file names, which

have the form:

oval_003.image

for image number 3 for example. There is an absolute limit of 40 characters

for IDENT. The extension (image in this case) is set using the EXTENSION

keyword.

The template keyword is an alternative to the IDENT keyword. In this case:

TEMPLATE oval_###.image

would work identically.

PROCESS gives the images to be generated, in this case from image 1 to image

1 (ie only one image is going to be examined), with a rotation angle of 1.0

degrees, starting at phi=0 (relative to the phi values given in

autoindexing). Note that for Mar Research, ADSC, RaxisIV and Mac Science

scanners, the phi values need not be given, they will be taken from the

image header.

DIRECTORY gives the directory name where the images are stored (up to 10

different directories can be given on one or more DIRECTORY keywords).

EXTENSION defines the extension of the image filenames (default is "image"

for Mar Research scanners, "osc" for R-axis, "ipf" for Mac Science, "cor"

for ESRF CCD, and "image" for unrecognised detectors.

DIVERGENCE... if two values are given, they are the horizontal and vertical

beam divergences (which will differ if a monochromator is used). Only a

single number need be given for isotropic divergence. "Horizontal" in this

context actually means in the plane containing the rotation axis and the

X-ray beam, which is horizontal in the case of the Enraf Nonius oscillation

camera and the Mar Research IP scanners, but vertical for standard Raxis

machines.

POLARISATION specifies the polarisation of the X-ray beam, and can be given

as PINHOLE or MIRRORS (both specify an unpolarised beam), MONOCHROMATOR (for

a GRAPHITE monochromator) or SYNCHROTRON followed by the degre of

polarisation (0.86 for SRS).

BEAM defines the direct beam position, in mm, relative to the position of

the first pixel in the image. This can be determined by taking a wax image

(or plasticine) and measuring the centre of the circles using the X-windows

display.

Special note for Raxis II scanners: The definition of the detector

coordinates in the R-axis software is different to that adopted in MOSFLM.

Thus if the direct beam coordinates have been obtained from the R-axis

software, then the X and Y coordinates must be interchanged.

BACKSTOP Defines the centre and radius of the backstop shadow. Reflections

lying within this circle will not be integrated. The position and size of

the shadow are best determined using the X-windows display.

GAIN defines the gain (adc units per X-ray photon) of the detector. This

should be constant for a given detector (and a fixed wavelength). It is

CRUCIAL to have a reasonable estimate of the gain as many aspects of the

program use counting statistics derived standard deviations to determine

acceptable spots for refinement, profiles etc. The correct value of the gain

should give a BGRATIO of unity (the BGRATIO is printed as part of the MOSFLM

output, as a function of intensity). The actual value of BGRATIO obtained

can be used to get an improved estimate of the gain using the relationship:

true gain = estimated gain * (bgratio)**2

The gain is typically between 1 and 2, but can be as high as 5 for Raxis II

detectors. See section 8.1 for a way of estimating the GAIN of your

detector.

DISTANCE is the crystal to detector distance in mm.

WAVELENGTH is the radiation wavelength (defaults to 1.5418).

HKLOUT gives the name of the output mtz file. This contains the Lp corrected

intensities and standard deviations, with the reflection indices reduced to

the asymmetric unit. This file can be used (after sorting) as input to the

CCP4 programs SCALA (or ROTAVATA and AGROVATA). HKLOUT can also be given on

the command line, but if both are given that specified by the keyword takes

precedence. If not given, the filename is made up from the image identifier

and the first image number, so in this case would be "oval_001.mtz".

SEPARATION (in mm in detector directions X and Y, ie horizontal and vertical

for the Mar IP scanner) gives the minimum allowed separation of two spots

before they are flagged as spatially overlapped (spots are treated as being

ellipsoidal in shape, the numbers given being the full axial lengths in the

X and Y directions). No attempt is made to integrate spatially overlapped

reflections. If not given, the program will work out suitable values based

on the spot size in the centre of the image. It will also determine if the

"CLOSE" option needs to be used (see SEPARATION in the helpfile for more

details). For spots that are not, in fact, completely resolved, the values

determined by the program may be too conservative and lead to a very large

number of spots being rejected as overlapped. In such cases, the SEPARATION

should be defined explicitly.

RASTER gives the parameters of the measurement box (see Fig. 1 above). As

explained in section 6.2.4 above, these values will be determined

automatically if not supplied.

GENFILE specifies the generate filename. If not given, it will default to

the MTZ filename, but with the suffix ".gen"

RESOLUTION: If not given, the resolution is set by the physical size of the

detector. Both high and low resolution limits can be given. A "dynamic" high

resolution limit, which depends on the mean I/sig(I), can be set using the

CUTOFF keyword. A specific resolution range can also be excluded (eg to

eliminate an ice ring) with the EXCLUDE keyword.

PLOT invokes the X-window interactive graphics option

RUN will start the processing

The image will be displayed with the predicted pattern overlaid. Fully

recorded reflections are displayed as blue boxes, partials as yellow boxes,

spatial overlaps as red boxes and reflections rejected as being too wide in

phi as green boxes. Clicking (left mouse button) on the centre of a box will

result in the reflection indices, phi value and phi width being displayed in

the "Output" window. If examining the image after integration, the profile

fitted intensity and standard deviation will also be given. See below for

more details on examining the image after integration.

The image should be examined carefully to ensure that the predicted pattern

actually matches what is on the image. If it does not, then any of the

relevant parameters (cell dimensions, missetting angles, beam divergence,

etc etc) can be adjusted and the pattern re-predicted (use the predict menu

item). A brief description of the various menu options is given below:

Predict If the cell dimensions or any other parameter in the "Display

Parameters" window is changed, selecting this menu item will re-calculate

the predicted reflections and display them.

Clear prediction This will delete the predicted pattern. It can be restored

by choosing the "Predict" menu item.

Adjust If there is an error in the beam coordinates or camera constants the

calculated pattern will be displaced relative to the observed pattern. The

"Adjust" option allows this to be corrected. The mouse is used to input the

calculated and the observed positions of two spots. From this the program

calculates the shift, rotation and scale factor required to superimpose

these two spots. The values of the shifts required are given and the user

given the choice of accepting the transformation or not.

Auto-refine This will refine the crystal missetting angles using the

AUTOMATCH option (see help library for more details), which essentially

adjusts the missetting angles to try to optimise the fit of the predicted to

the observed pattern. This can converge from initial errors of 1-2 degrees,

but the final parameters are NOT as accurate as those obtained from

post-refinement. With the use of REFIX, this option should not normally be

necessary. Note that it will use the default parameters for the refinement

(eg only using data to 6A). You may wish to modify these using the

appropriate keywords...see help library documentation. After refinement it

will go on to measure the image. **** THIS OPTION IS NO LONGER SUPPORTED

****

Integrate This will close down the display and measure the image in the

normal way. You are given the option of displaying the image again after

positional refinement has been carried out, or after the integration has

been carried out. (See below)

Find hkl If this menu item is activated, the user is prompted for the hkl

indices of the spot he wishes to find. A blue cross is drawn on the position

of the spot. A warning is given if the spot does not lie in the displayed

part of the image. This can be useful in identifying bad spots.

Read spot list This allows a list of spots produced by using the "save

spots" option to be displayed superimposed on the image. It is essentialy

obsolete now. Note that ONLY spots from a single image (ie up to the first

terminator (-99 or -999) in the spots file will be displayed).

Pick This will display the actual pixel values in a box around the cursor

position (the size of the box can be set in the "Display Parameters" window.

Measure Cell Allows measurement of cell parameters, but the distance and

wavelength must be set correctly.

Circles Puts up resolution circles

Fit circles Will determine the centre of a circle defined by clicking with

the mouse at a series of points lying on the circle. After selecting "Fit

circles" select a series of points with the mouse and then selet "Fit

points". The rms fit, radius and centre of the circle will be given. The

user has the option to update the direct beam coordinates to the circle

centre. Used to determine the direct beam position from wax rings.

EXIT Closes down the display. If the image was being displayed with the

IMAGE keyword, the mosflm prompt will return. If all keywords for processing

are given, then MOSFLM will proceed to measure the image(s).

Examining the image after integration

There is the option to update the display after integration (At the bottom

of the "Processing Parameters" window, this is one of a number of On/Off or

Yes/No toggles). If this option is chosen, once the profiles have been

determined and the image integrated, the image plus the predicted pattern

will again be displayed again. (Note that there is an overhead associated

with this, because the image will have to be read into memory again (unless

only a single image is being processed)).

If there are any "bad spots" (ie poorly measured reflections) on the image a

window will be displayed which gives the user has the option to examine

and/or edit the badspots. If this option is selected, the image will be

displayed with a new menu option "Bad spots". Bad spots can then be

reclassified as acceptable and other (accepted) spots can be re-classified

as rejected.

In addition to the predicted reflections, vectors will be drawn indicating

the difference between the predicted and observed spot positions for FULLY

RECORDED reflections (these vectors are in red....it may be necessary to use

the "Clear Prediction" menu option to see them clearly. The vectors can be

scaled using the "Vector scale" menu item in the Processing parameters

window. Also a minimum intensity threshold for the display of these vectors

can be set using the "Threshold" menu item.

The vectors will of course be longer for weak spots than strong ones, but

for all reflections the direction of these vectors should be random. If this

is not the case, it suggests errors in crystal cell parameters or

orientation, or misclassification of fully recorded/partially recorded

reflections, or the existence of spatial distortion which is not being

correctly modelled.

"Badspots" will be indicated as red crosses, rejected reflections as blue

croses. Rejected reflections will normally be those containing a zero pixel

value (because the measurement box extends outside the scanned area of the

image plate); these are NOT classified as "badspots". They may also arise if

a very large number of background pixels have been rejected.

Overloaded reflections will be indicated as green crosses. Note that you

MUST include keywords PROFILE OVERLOAD in order to estimate the intensities

of overloaded reflections by profile fitting.

By clicking on a reflection, the LP corrected profile fitted intensity and

standard deviation will be given in the output window.

----------------------------------------------------------------------------

6.3.2 Processing the first block of data (Non-interactively)

It is usually advisable to process a block of (say) 10 degrees of data prior

to processing the complete dataset (as this is quite time consuming) just to

check that the processing is satisfactory.

The commands for MOSFLM might now be:

TITLE Procesing test data

! Crystal parameters

MATRIX oval_2_3.mat

SYMMETRY 96

MOSAIC 0.1

! image parameters IDENT oval

PROCESS 1 TO 10 [ ANGLE 1.0 START 0.0 ]

DIRECTORY /scr0/andrew/

EXTENSION image

! beam parameters

DIVERGENCE 0.35 0.2

POLARISATION MIRRORS

! detector parameters

BEAM 89.33 90.10

BACKSTOP CENTRE 88.5 91 RADIUS 14

GAIN 1.2

DISTORTION ROFF 0.3 TOFF 0.1

! The following are read from the image header if not supplied on

! keywords for Mar, ADSC, RaxisIV and Mac Science. The phi values (on

! PROCESS keyword) will also be read from the header if not supplied.

DISTANCE 124.1 WAVELENGTH 1.542

!The following are optional, if not given, the program will set

!suitable defaults.

!HKLOUT oval.mtz

!SEPARATION 1.3 1.3

!RASTER 17 17 9 3 3

!GENFILE oval_1to1.gen

!RESOLUTION 3.0

PLOT

RUN

See above (6.3.1 Running MOSFLM interactively) for a description of each of

the keywords.The only difference to the commands described above is that the

PROCESS keyword has been set up to process the first 10 images. The ADD

subkeyword on the PROCESS line specifies that the batch number on the output

mtz file should be (1000+image number) (ie 1001-1010 in this example).

DISTORTION specifies the ROFF and TOFF values for this scanner. Note that it

is now possible to specify the radially dependent radial and tangential

offsets using sub-keywords RDROFF, RDTOFF respectively (See Appendix IV). It

is not normally necessary to specify these values unless thay are large

(greater than 0.3).

The program will then form the standard profiles by summing reflections over

the first block of images 1 to 5 and print the resulting profiles. The

number of images in a block is set by the program, but may be set explicitly

by the PROFILE BLOCK keywords).

6.3.2.1 Formation of the Standard Profiles

The standard profiles are determined in a number of areas across the

detector. By default the detector is divided into 9 regions for data to a

resolution lower than 2.5A and 25 regions of which only 22 lie within the

active area of a circular detector for resolution higher than 2.5A.

Alternatively the user can define the set of lines parallel to the detector

X and Y axes which define the standard areas. This is done with keyword

PROFILE XLINES... YLINES...

EXAMPLE:

PROFILE XLINES 0 45 90 135 180 YLINES 0 45 90 135 180

will divide the a detector 180x180mm into 16 areas. See MOSFLM Help file for

more details.

A separate standard profile is evaluated for each of these areas.

The program prints out some statistics on the standard profiles, followed by

statistics on profiles that it has averaged (if any) and followed by a

representation of each of the standard profiles using a single character (0-

9, then A-Z) to represent the value at each pixel (A [ denotes a negative

value). In these representations, a minus sign denotes the background

region, and a * denotes rejected pixels. Background pixels which are

overlapped by the peak regions of neighbouring spots are automatically

rejected by the program. It will also warn you if the peak regions of

neighbouring spots overlap.

6.3.2.2 Inadequate profiles

Each standard profile has to satisfy two criteria before it is considered

acceptable. There must be at least ten contributing reflections, and the rms

variation in the background plane (after rejecting outliers) must be less

than 10 (after scaling the profile to a maximum value of 255). If a profile

fails to pass either test, then it is averaged using the profiles from

neighbouring areas on the detector. Profile averaging should be avoided if

at all possible. The averaging inevitably produces a profile that is less

broad than the original profile because it is dominated by the stronger,

lower resolution data. Look at the printed profiles before and after

averaging to confirm this.

Accumulating the profiles over a BLOCK of say 10 images (see below) should

help provide a sufficient number of reflections, but is unwise to accumulate

over too wide a phi range because this will average out any genuine

variation in the profiles with phi (eg due to a change in effective

diffracting volume). Both rejection criteria can be changed using

subkeywords (NREF for number of reflections, RMSBG for rms variation in

background, on the PROFILE keyword line) and it is usually better to avoid

averaging by changing these criteria if necessary:

PROFILE RMSBG 20 NREF 5

6.3.2.3 Spots running into each other

The program will give a warning if it detects that the peak areas of

adjacent spots overlap. There are two possible ways around this:

1) Increase the SEPARATION parameters. Making the separation significantly

smaller than the actual spot size in the centre of the image can lead to

serious problems and is NOT recommended.

2) The actual spot size that the program works with when testing for peak

overlaps (after rejecting those that are too close as determined by the

SEPARATION parameters) is determined by the "profile optimisation"...that is

when the program works out the best values of the measurement box parameters

NC, NRX, NRY, which is done independently for each of the standard profiles.

If there is significant diffuse scatter on the image, the "optimised" raster

parameters may well produce a peak area that is actually slightly broader

than the true Bragg peak and includes part of the "diffuse" peak. This can

be checked by examination of the standard profiles...if the peak area

contains many pixels with values of 0 or 1 then it suggests the peak is too

broad. This effect can be overcome by increasing the TOLERANCE parameter

(See the help library for more details on how the optimum parameters are

derived and the effect of the TOLERANCE parameter). The default value for

this parameter is 1% ie 0.01. Increasing it (try steps of 0.005, but it

should not be necessary to go above 0.04) will result in a reduction of the

optimised peak size. It is up to you to decide what the optimimum value is

on the basis of the appearance of individual spots in the image.

6.3.2.4 Output Statistics

The other statistics produced are for general information, and are described

in the MOSFLM help library under "Output". Probably the most useful is the

breakdown of I/sig(I) as a function of resolution. This will give an

immediate idea of the quality of the data...particularly at the high

resolution end. For guidance, a mean I/sig(I) of 3.0 will give an R-merge of

between 20% and 30% in SCALA. If there are symmetry related fully recorded

(or summed partial) reflections on a single image, statistics are also

provided on the agreement between their intensities.

Check the following in this job:

1) Check the standard profiles look OK (ie the peak is within the peak

region).

2) Check the weighted residual is about 1.0.

3) CHECK FOR WARNING MESSAGES. These are given at the end of the logfile and

in the summary file. They will point out possible problems and suggest a way

around them.

----------------------------------------------------------------------------

6.3.3 Finally, Processing the dataset

The whole philosophy of MOSFLM is to allow the entire dataset (or all images

obtained from a single crystal) to be processed in a single job. To make

this possible, the crystal orientation can be refined continuously for every

image, to take account of possible crystal slippage, and the cell parameters

can also be refined if the initial estimates are not accurate. An accuracy

of 1 part in 1000 or better is required for optimal processing of high

resolution data.

There are a large number of adjustable parameters within MOSFLM, but

considerable effort has gone into making the program select an appropriate

value for these parameters. The program defaults should therefore ALWAYS be

used unless there is a very specific reason for changing parameters (eg if

it is suggested in the warning messages in the summary or logfile)

See section 9.3 for a complete example command file.

----------------------------------------------------------------------------

Back to Contents Page

7: Interpreting the output

7.1     The log files

7.2     The summary file

7.3     Checking the quality of the data

----------------------------------------------------------------------------

7.1 The log files

The MOSFLM log files can be very long, and to simplify assessing the

performance of the processing the program writes a summary file which

contains most of the important information.

However the initial part of the logfile which gives information on the

parameters used in the processing (along with the keywords used to change

these parameters) should ALWAYS be read.

The standard profiles should also ALWAYS be checked to ensure that the

background mask optimisation has worked correctly (particulary if there is a

high degree of diffuse scatter or the spots are very close).

Warning message are written to the end of the logfile and the summary file

if the program detects possible problems. These messages should be acted on

where appropriate.

7.2 The summary file

A graphical representation of the information in the summary file can be

obtained by running the CCP4 program "xloggraph" (followed by the name of

the summary file).

The information contained in the summary file is listed below:

7.2.1 Statistics on Processing

IMAGE

The image number (as given on the PROCESS keyword)

CCX,CCY,CCOM

The camera constants (in mm and degrees). The magnitude of CCX and CCY

reflects the accuracy of the supplied direct beam coordinates. However their

actual magnitude is really immaterial (providing they do not exceed your

expected error in the direct beam coordinates) but they should remain

CONSTANT throughout the data collection (assuming the beam is not moving !).

If using the post-refinement option to refine crystal missetting angles,

then CCOMEGA may show a gradual drift, since any errors in orientation

around the direct beam are taken up by CCOMEGA.

DIST

The refined crystal to detector distance.

YSCALE

The relative scale factor in the detector Y direction. This should be very

close to unity (to within 1 part in 1000 or better). Deviations from unity

indicate errors in cell parameters. Note that for the R-AXIS detector the

difference in the pixel sizes in the X and Y directions is taken up by

YSCALE, which should have a value close to 1.032.

TILT,TWIST

Deviations from normal incidence on the detector. TILT is a rotation about a

horizontal axis, TWIST about a vertical axis, and the values given are

expressed in hundredths of a degree. These values should be close to zero

(less than 20) and should be CONSTANT (to within experimental error).

ROFF,TOFF

The radial and tangential offsets for the Mar Research and Mac Science

scanners only (see Appendix IV) in mm. These should be CONSTANT to within

experimental error. Watch out particularly for variations in ROFF by one

pixel. This indicates that the limit switch on the scanner is not

functioning correctly (this should only occur very rarely if at all). A

Warning is given if this happens.

RESID

The rms positional residual (in mm) after refinement of the detector

parameters. For strong images this should be between 0.02 and 0.04. For weak

images or large spots (due to high mosaic spread) it can be significantly

higher. If partials have to be included in the refinement it will also be

higher. A dataset with rms residuals of 0.2 to 0.3 can still give a final

Rmerge of under 10% ! Remember a residual of 0.15 is still only one pixel.

If the residual is greater then 0.04 for a strong image, there is almost

certainly an error in the cell parameters, and they should be refined using

the POSTREFINEMENT options. Even for weaker images, the positional residual

for the initial refinement (using only central spots) should be small.

WRESID

The weighted residual. This should be close to unity (independent of the

strength of the image). Larger values suggest errors in cell parameters or

crystal orientation.

FULL,PART,OVRL,NEG

The number of fully recorded, partially recorded and overloaded reflections

measured on the image (this includes reflections with part of the

measurement box outside the scanned region...these are not actually

integrated unless the PROFILE EDGE keywords are included, in which case

those with at least half of the pixels inside the scanned area will be

measured by profile fitting). Overloaded reflections are also not integrated

(by default) but if the PROFILE OVERLOADS keywords are included, then their

intensity will be estimated by profile fitting the non-overloaded pixels.

NNEG is the number of reflections with negative (summation integration)

intensity. The proportion of the dataset that falls into this category

obviously depends on the strength of the images.

BAD

The number of badspots. Reflections will be classified as badspots if they

fail any of five criteria: BGRATIO greater than 3, PKRATIO greater than 3.5

(Fully recorded reflections only, and you have the choice of either

rejecting the reflection completely (this is the default) or setting the

profile fitted intensity and sd. to the summation integration values

(REJECTION PKRATIO 3.5 ACCEPT)...see help library), intensity negative and

greater (absolute value) than 5 sigma, background gradient too large

(background gradient/ average background exceeds 0.03), too few background

pixels remaining after rejecting outliers (less than 10). All these criteria

(except negative intensity) can be changed using the REJECTION keyword.

There should be less than 5-10 badspots on any one image. If there are more

it suggests problems. The badspots are listed in the logfile with the

information necessary to see why they have been flagged. It may also be

helpful to have the pixel values of flagged spots. These can be obtained by

using the keywords: REJECTION PLOT. This should enable the source of the

problem to be identified.

I/SIGI (two columns)

This gives the average I/sd(I) for the whole dataset (first column) and the

outermost resolution bin (second column).

Rsym

This gives the R-factor (on intensities) for symmetry related fully recorded

(or summed partial) reflections on the same image.

Nsym

The number of reflections (not the number of observations) included in Rsym.

SDRAT

The ratio of the observed agreement between symmetry related reflection

intensities to their estimated standard deviations. This should have a value

of 1.4 if there are two measurements of each reflection, or very close to

unity if four or more. This can be more useful than the Rsym value, as it

should not depend on the intensity of the measurements while Rsym will

always be higher for weak spots.

7.2.2 Results of the post-refinement

These should be self-explanatory. Check that any change in missetting angles

is gradual. Changes greater than about one fifth of the sum of the mosaic

spread and beam divergence (ie typically 0.05 degrees for a good crystal)

per image will give rise to errors in the intensities of partially recorded

reflections if multiple oscillations have been used in the data collection.

There is no way of correcting for this. Changes in cell parameters (if

refined) should really not occur (unless the cell genuinely changes with

radiation damage). If they do, consider increasing the number of images used

in postrefinement (ADD/WIDTH). Ideally there should be a data to parameter

ratio of 10:1 in the post refinement. If there is not, consider reducing

SDFAC which sets the I/sigma(I) criterion for selecting reflections. The

refined angular residual should be about one tenth of the summed mosaic

spread and beam divergence, although this will depend on the strength of the

reflections included in refinement. If the beam parameters are refined,

check that they are stable, particularly if they are being used in the

reflection list generation (USEBEAM). If they are not stable, take an

average value and use that in MOSFLM and do NOT use the refined values in

generating the reflection list within MOSFLM (ie do NOT include the USEBEAM

subkeyword).

7.3 Checking the quality of the data

Although there are indicators of data quality in MOSFLM (in particular the

I/sig(I) and Rsym values as a function of resolution), the only satisfactory

way of assessing data quality is to look at the results of merging

measurements of symmetry related reflections using SCALA or AGROVATA.

Remember that the R-factor alone is not a good indicator, it will always be

high for weak data. What is probably more important is the standard

deviation analysis at the end of SCALA or AGROVATA. If this suggests that

the observed agreement is that expected based on the standard deviations (ie

SIGM is 1.0) then you cannot hope to do any better. Inevitably there are

errors which are not accounted for in the estimated standard deviation. Thus

it is quite normal to have to boost the standard deviations by 20-30% (ie an

SDFAC of 1.2-1.3) to achieve a SIGM value of 1.0. In addition, the agreement

is generally worse for the strongest data, so an SDADD of between 0.02 and

0.03 is quite common. If these parameters have to be made significantly

greater than this to achieve a SIGM value of 1.0 across the intensity range,

then this indicates problems with the processing which should be

investigated. One possibility is the presence of a few large outliers, which

can destroy the SIGM analysis...look at the monitored reflections for

evidence of this. In cases where the crystal has a high mosaic spread, pay

particular attention to the partial bias analysis. If this is more than

1-3%, then the mosaic spread has probably been incorrectly defined. In

difficult cases it may well be necessary to process a dataset several times

(eg with different mosaic spreads, or different numbers of images included

in post refinement or in forming the standard profiles) in order to achieve

the best final dataset, but this should be simply a case of using more cpu

time, and not require a lot of intervention.

Finally, always consider the possibility that the spacegroup is incorrect !

----------------------------------------------------------------------------

Back to Contents Page

8: General tips

8.1     Estimating the GAIN of a detector

8.2     Processing images with no (or very few) fully

recorded reflections

8.3     Processing images when the spots are not fully resolved

8.4     Processing data from other detectors, or standard

detectors with different rotation axis orientation.

----------------------------------------------------------------------------

8.1 Estimating the gain of the detector.

The detector GAIN is the factor that converts counts in the digitised image

into the equivalent number of absorbed X-ray photons. It is used to estimate

standard deviations, reject outliers etc.

Thus: (value in digitised image) = GAIN * (Equivalent number of photons)

The simplest way to get an estimate of the gain is to display an image using

the IMAGE keyword. Select an area which is free of any diffraction spots,

but has a reasonable number of background counts (ie at least 100 per pixel,

say). Drag out a small rectangle in this "spot free" area with the mouse.

Then within the "Output" window of the display you will find the entries:

Average 335.5 Rms 19.1 Number 345

(The values will be those for your image of course). Try to get an area with

at least 100 pixels ("Number" is the number of pixels in the area you have

dragged out), but don't make it too large because what you are looking for

here is a "flood field", ie an area within which you have got a uniform

background.

The GAIN can then be estimated as:

GAIN = (rms*rms)/Average

For the example numbers above this would be 1.09

Try this for a few areas and choose the LOWEST value you get (any features

in the background, such as diffuse scatter, a weak spot etc, will INCREASE

the rms, but nothing will DECREASE it).

( NOTE: This procedure assumes that the counts in each pixel are

independent. For some detectors this will NOT be the case, particularly if a

small pixel size (100 microns or less) is used with a standard image plate.

In these circumstances this will give an UNDERESTIMATE of the true GAIN.)

This should give a reasonable starting value. If you then process some

images, the program calculates a parameter BGRATIO which is the ratio of the

observed rms variation in the background around spots to what is expected

from counting (Poisson) statistics based on the supplied value of the gain.

This ratio should be 1.0 if the GAIN is correct (and providing the spots are

all contained within the peak areas, ie there is no "diffuse scatter halo"

surrounding the spots).

If BGRATIO differs from 1.0 by more than 10%, the program gives a "Warning"

message. The correct gain can then be estimated as: TRUE GAIN = (INPUT

GAIN)*BGRATIO*BGRATIO

8.2 Processing images with no (or very few) fully recorded reflections

If less than one third of the total number of reflections predicted is fully

recorded (this value can be changed with keywords REFINE FULLFRAC) then

partially recorded reflections are automatically included in the refinement

of the detector parameters.

When adding partials (ADDPART option, this is NOT the default and must be

requested by giving the keyword ADDPART) then only partials at the end of

the oscillation range will be selected and the other "half" of the partial

will automatically be added in from the next image, so the centroids of

these reflections will be determined as accurately as those of fully

recorded reflections (provided the detector origin is indeed fixed).

If this is not the case (ie by default), partials from both the beginning

and end of the oscillation will be selected but only if their degree of

partiality is greater than 0.8 (this limit can be changed by giving an

appropriate value after the subkeyword PARTIALS eg REFINEMENT INCLUDE

PARTIALS 0.5 will include any partial more than 50% recorded). If

insufficient reflections are found for refinement, the limit on the degree

of partiality will be relaxed until sufficient reflections are found.

Similarly, if less than one third of the total number of reflections

predicted is fully recorded, partials will also be included in forming the

standard profiles. Providing the profiles are being accumulated over a

number of images, this will not introduce a significant error as both parts

of most reflections will be included, so the fully recorded profile will be

generated. The inclusion of partials in the standard profiles can be

prevented by keywords: PROFILE FULL.

8.3 Processing images when the spots are not fully resolved

When adjacent spots are very close the SEPARATION CLOSE option for

integration should be used (see the help library for details of what this

option involves). The program will automatically invoke this option if the

SEPARATION keyword is NOT given and the closest possible spot separation

(for any orientation of the crystal) is such that adjacent spots are not

separated by more than 2 pixels.

REMEMBER that when using the CLOSE option, a scratch file (COORDS) is used

and this file MUST be unique (ie you cannot run two jobs from the same

directory unless COORDS is assigned to a unique filename).

Spots which are not completely resolved on the detector or high levels of

diffuse scatter may result in a larger peak region in the measurement box

than is desirable, due to the algorithm used in optimising the background

raster parameters (see section 6.2.4). If the peak region includes too much

of the broad diffuse peak under the Bragg reflection or if it includes the

"tail" of a neighbouring spot then it can be made tighter by adjusting the

TOLERANCE. Try increasing the "inner" tolerance first,

eg PROFILE TOLERANCE 0.02 0.03

It may also be necessary to specify the minimum spot separation explicitly:

eg SEPARATION 0.95 0.95 CLOSE

The spot separation can be made as small as the spot size in the centre of

the detector (even though the spots are always larger at the outside of ther

detector due to oblique incidence).

In severe cases, it may be worth trying to reject pixels around the edge of

the peak if they do not fit the profiles well (because there is a

neighbouring very strong peak). See keyword PROFILES subkeywords WDLIM1

WDLIM2 in the help library for more information.

8.4 Processing data from other detectors

If a commercial detector is being used on a synchrotron beamline, it is not

uncommon for the direction of rotation to be different to that used on the

commercial instrument. This will not affect autoindexing from a single

image, but the resulting orientation will not correctly predict the next

image. To correct for this, use the keywords:

DETECTOR REVERSEPHI

If the orientation of the rotation axis has changed by 90 degrees (eg

horizontal rather than vertical) this can be allowed for by redefining the

OMEGA angle (see Appendix IV), using keywords:

DETECTOR OMEGA

For example, the value for OMEGA is 90 degrees for a Mar IP scanner, but if

it was being used with a vertical rotation axis of a goniostat, you would

need to give the keywords:

DETECTOR MAR DETECTOR OMEGA 0

Note that for RAXIS, DIP and LIPS IP scanners there is a specific subkeyword

to specify the orientation of the rotation axis.

Default values of OMEGA are 90 for Mar, Raxis IP scanners, 180 for DIP2000

or 2020 or 2030 (assumed to have a horizontal rotation axis), 270 for the

ESRF CCD detector (with image intensifier). If you want to know what the

default value of OMEGA is for a given type of detector, add the keyword:

DEBUG CONTROL.

Then in the output file immediately after reflecting the DETECTOR keyword

the following line of debug will appear:

Machine type: RAXI Model type: RAXISIV INVERTX F SPIRAL F ORTHOG T CIRCULAR

F OMEGA 180.0

It should be possible to process data from other detectors but, depending on

how the images are written, it may be necessary to write new code to read in

the image. See keyword DETECTORS in the help library for further details.

----------------------------------------------------------------------------

Back to Contents Page

9: example command files

9.1     Autoindexing an initial image (interactively)

9.2     Determining an accurate cell

9.3     Integrating a series of images

----------------------------------------------------------------------------

There follow a number of example command files. In general, it is best to

let MOSFLM choose appropriate default values for processing, and to only

specify additional parameters if you get warning messages suggesting changes

(these warning messages are given in the Summary file and at the end of the

logfile).

It is usually NOT a good idea to start with someone elses command file, as

they have have set some parameters which are not appropriate for your data.

9.1 Autoindexing an initial image

This will normally be done interactively. The commands can be put in a file

(eg "runit") and executed by typing @runit at the MOSFLM prompt.

=> TITLE My lysozyme data               ! This title is transferred to the

! MTZ file

=> IMAGE lyso_001.image [PHI 0 TO 1]    ! Filename of first image. For Mar,

! ADSC, Mac Science and RaxisIV images

! the phi values will be taken from

! the image header if not given here.

=> BEAM 150.0 149.0                     ! Direct beam coordinates

If not a Mar Research IP scanner:

=> DETECTOR RAXISIV                      ! or RAXIS (for RAXIS II) or DIP2000 etc

If not processing Mar Research, R-axis or Mac Science images:

=> WAVE 0.91                            ! For Mar, ADSC,RaxisIV and Mac Science this

=> DISTANCE 300                 ! information is taken from the header

! but can be overwritten using the

! keywords.

=> SYMM p43212                  ! If known, give cell and symmetry

=> CELL 79 79 38                        ! otherwise omit completely.

Not essential for first stages, but needed for integration

=> DIVERGENCE 0.1 0.03          ! If isotropic, the beam divergence

! can be included in the mosaic spread.

=> SYNCHROTRON POLARIZATION 0.9 ! Defaults to 0.86 (SRS, Daresbury UK)

=> GAIN 1.7                             ! See section 8.1 for a way to

! estimate the gain if not known.

=> GO

Further options are then usually invoked via the menu of the X-window

interface.

9.2 Determining an accurate cell

This example assumes that an orientation matrix has been obtained for the

first image, and that accurate cell parameters are to be determined using

two (one can use one or more) segments of data. Note that this can be done

interactively via the X-windows menu, the example given below is for a

background job.

ipmosflm spotod /scr0/andrew/f1adpx3.spotod \

<< eof-ipmosflm

TITLE Postrefining the cell with two segments

! Source parameters

WAVE 0.91                       ! For Mar, ADSC, RaxisIV and Mac Science

! this information is taken from the

! header but can be overwritten using

! the keywords.

SYNCHROTRON POLARISATION 0.86   ! Default polarisation is 0.86 (SRS)

DIVERGENCE 0.10 0.02            ! Horizontal and vertical divergence

DISPER 0.00020                  ! Wavelength dispersion

! Detector parameters

DETECTOR RAXISIV                 ! If not a Mar Research IP scanner:

! can be  RAXIS (for RAXIS II), DIP2000 etc

BEAM 150.0 149.0                ! Direct beam coordinates

GAIN 1.2                        ! Detector gain

DISTANCE 300                    ! For Mar, ADSC, RaxisIV and Mac Science

! thisinformation is taken from the

! header but can be overwritten using

! the keywords.

BACKSTOP CENTRE 148 151 RADIUS 12       ! Beamstop shadow

! Crystal parameters

SYMMETRY P212121

MATRIX image_001.mat            ! orientation matrix for first image

MOSAIC 0.22                     ! Mosaic spread, should be a reasonable estimate

! Image parameters

IDENT f1adpx3                   ! Sets template for image filenames

DIRECTORY /scr1/andrew/images/  ! Directory where images are stored

! Processing parameters

POSTREF SEGMENT 2               ! postrefine using two segments

PROCESS 1 to 4 [START 0 ANGLE 1]  ! Images to be used in first segment.

! For Mar, ADSC, RaxisIV and Mac Science

! the phi values are taken from the

! header but can be overwritten using the

! keywords.

NEWMATRIX f1adpx3_postref.mat   ! Filename for output orientation matrix

! This will contain the refined cell

! and the refined missetting angles for

! the first image (1 in this case).

GO

PROCESS 86 to 89 [START 85 ANGLE 1.0]   ! Images to be used in second segment

! MATRIX image_086.mat          ! If necessary (ie crystal has slipped) can

! specify an orientation matrix for the first

! image of the second run.

GO

eof-ipmosflm

Possible additional keywords:

1) If there is significant diffuse scatter in the image, or if the mosaic

spread is very large (greater than 0.5 degrees) it is usually best to limit

the post-refinement to using reflections that are nearly half-recorded,

using the FRMIN, FRMAX keywords. This will make the refinement less

dependent on the model of the rocking curve:

eg POSTREF FRMIN 0.4 FRMAX 0.6

2) If there is an ice ring or spots on the image, all spots within a

specified resolution limit can be rejected.

eg RESOLUTION EXCLUDE 3.66 3.72

9.3 Integrating a series of images

This example assumes that an accurate cell has already been obtained (using

a POSTREF SEGMENT run) so no further refinement of cell parameters is

required. Note that integration can be done interactively via the X-windows

menu, the example given below is for a background job.

ipmosflm spotod /scr0/andrew/f1adpx3_1to90.spotod \

summary f1adpx3_1to90.sum \

coords /scr0/andrew/f1adpx3_1to90.coords \

<< eof-ipmosflm

TITLE Postrefining the cell with two segments ! Source parameters

WAVE 0.91               ! For Mar, ADSC, RaxisIV and Mac Science this

! information is taken from the header

! but can be overwritten using the

! keywords.

SYNCHROTRON POLARISATION 0.86 ! Default polarisation is 0.86 (SRS)

DIVERGENCE 0.10 0.02      ! Horizontal and vertical divergence DISPER 0.00020

! Wavelength dispersion

! Detector parameters

DETECTOR RAXISIV           ! If not a Mar Research IP scanner:

! can be RAXIS (for RAXIS II), DIP2000 etc

BEAM 150.0 149.0          ! Direct beam coordinates

GAIN 1.2                  ! Detector gain

DISTANCE 300              ! For Mar, ADSC, RaxisIV and Mac Science this

! information is taken from the header

! but can be overwritten using the

! keywords.

BACKSTOP CENTRE 148 151 RADIUS 12    ! Beamstop shadow

! Crystal parameters

SYMMETRY P212121

MATRIX f1adpx3_postref.mat ! orientation matrix previous job.

MOSAIC 0.22                ! Mosaic spread, should be a reasonable estimate

! Image parameters

IDENT f1adpx3                      ! Sets template for image filenames

DIRECTORY /scr1/andrew/images/     ! Directory where images are stored

! Processing parameters

POSTREF FIX ALL            ! do not refine cell, only crystal orientation

PROCESS 1 to 90 [START 0 ANGLE 1] ADD 1000 ! Images to be integrated.

! For Mar, ADSC, RaxisIV and Mac Science

! the phi values are taken from the header

! but can be overwritten using the

! keywords. The batch numbers in the output

! MTZ file will be the image number plus 1000

! (ADD keyword).

ADDPART                       ! Requests addition of partially recorded

! reflections over two images. This

! should only be used if the incident flux is

! constant (or the exposure is flux

! controlled) and the detector is mechanically

! stable.

! See section 6.2.2

HKLOUT f1adpx3_1to90.mtz      ! Name of output MTZ file.

GO

END

eof-ipmosflm

# Delete temporary files

/bin/rm /scr0/andrew/f1adpx3_1to90.spotod

/bin/rm /scr0/andrew/f1adpx3_1to90.coords

/bin/rm f1adpx3_1to90.gen

Possible additional keywords:

PROFILE TOLERANCE 0.02 0.03

PROFILE XLINES 0 75 150 225 300 YLINES 0 75 150 225 300

SEPARATION 0.75 0.75 CLOSE

AUTOINDEX

CELL KEEP REJECTION PKRATIO 4.0

RESOLUTION EXCLUDE 3.66 3.72

TWOTHETA 15

BEAM SWUNG_OUT 99.96 124.5

LIMITS RSCAN 147.0

These are described below.

1) Optimising the Standard Profiles

Depending on the strength of the images, the degree of diffuse scatter, the

spot separation on the images, the crystal mosaicity etc, it may be

necessary to adjust the PROFILE TOLERANCE parameters to get well defined

standard profiles. The appearance of the standard profiles should always be

checked in the logfile, to ensure that adjacent spots are not included in

the "peak" region, that long diffuse tails are not being included in the

peak, and that not too many profiles are being averaged (see below).

eg PROFILE TOLERANCE 0.02 0.03

The first value is used for profiles near the centre of the image, the

second value for profiles at the outside, and an interpolated value for

profiles inbetween. Defaults are 0.01 0.01 for a lab source (wavelength

1.542) and 0.01 0.03 for a synchrotron.

It should not normally be necessary to use values above 0.04.

For weak images, you may find that some profiles are being averaged. This is

to be avoided if possible. Consider if you are trying to integrate beyond

the true resolution limit of the crystals. If not, try to avoid the

averaging by one of the following:

i) If profiles are being averaged because there are very few reflections,

but the reflections are reasonably strong so that the profiles look OK,

reduce the minimum number of reflections required (default 10): eg PROFILE

NREF 5

ii) If the profile is being averaged because the rms variation in the

background (after scaling the peak to 255) is too large, but in fact this is

because there is significant diffuse scatter which should not be included in

the Bragg peak, then increase the allowed rms variation (default 10.0): eg

PROFILE RMSBG 20.0

iii) Try setting up fewer standard profiles, by defining the regions on the

detector where the profiles are to be set up using the XLINES,YLINES

keywords:

eg PROFILE XLINES 0 75 150 225 300 YLINES 0 75 150 225 300

will give 16 areas over which a profile will be determined.

2) Specifying the minimum spot separation

The program will automatically determine the minimum allowed spot separation

based on the size of spots in the centre of the first image to be processed.

Spots closer than this will not be integrated. However, if the spots are

very close together, the minimum spot separation determined by the program

may be too conservative and result in many spots being rejected. To avoid

this, set the minimum spot separation explicitly. Note that in such cases

the "CLOSE" option for spot integration should be used (see the Help library

under "SEPARATION" for further details of this option). The program also

decides if the "CLOSE" option need to be invoked, again based on the very

first image to be processed. It is a good idea to ensure that if the CLOSE

option is used for one segment of data (eg the 90 images processed in the

job above) then it is also used for all other data from this crystal, by

specifying its use explicitly.

eg SEPARATION 0.75 0.75 CLOSE

or if just wanting to enfore the use of the CLOSE option:

SEPARATION CLOSE

3) Refining cell parameters during integration

In this case, replace the POSTREF FIX ALL keywords with

POSTREF WIDTH 10

where WIDTH specifies the width (in phi) of data to be used in the

refinement. For trigonal or higher symmetry a few degrees of data is usually

sufficient, for lower symmetries at least 10 degrees is usually necessary.

THIS IS NOT RECOMMENDED: It is usually preferable to determine the cell

initially using the POSTREF SEGMENT option.

4) Determining the orientation as part of the job

It is possible to invoke the autoindexing as the first part of a processing

job. By default the indexing will be done on the first image to be

processed, but the images to be used can be specified explicitly (See

section 3.2). In these circumstances the unit cell derived from the

autoindexing will be used in the integration, unless the KEEP subkeyword is

given with the CELL. Usually an accurate cell (from post-refinement) will be

available and should be used in the integration:

AUTOINDEX [ IMAGE 1 2 3 ]

CELL KEEP 283.09 107.60 139.65 90.000 90.000

90.000

5) Specifying the size of the integration box

The overall dimensions of the integration or measurement box are determined

automatically based on strong spots from the first image to be processed. If

you want to make the box smaller or larger than this, specify its size

explicitly and tell the program not to change it:

RASTER 25 25 17 3 3

PROFILE FIXBOX

The corner and rim parameters (17 3 3 in above case) will still be

optimised, but the overall dimensions will not (use PROFILE NOOPTIMISE

(ATALL) to suppress optimisation).

6) Excessive numbers of "Bad spots"

Excessive numbers of "Bad spots" is usually a sign that the processing is

not as good as it should be and there are errors in the cell parameters,

crystal orientation, mosaic spread or in the detector itself. However, if

the cause of the bad spots cannot be corrected, it may be preferable to

change the rejection parameters to avoid too many reflections being

rejected. This is particularly true if a large number of strong reflections

are being rejected because of a poor profile fit, because this could have a

significant effect on Patterson based methods used in Molecular Replacement,

definition of solvent masks etc.

eg REJECTION PKRATIO 5.0

This should ONLY be done as a last resort, it is much better to find the

cause of the rejections. See REJECTION keyword in Help library for more

details.

7) Ice rings

If there is an ice ring or spots on the image, all spots within a specified

resolution limit can be rejected.

eg RESOLUTION EXCLUDE 3.66 3.72

8) Inclusion of partially recorded reflections in refinement and profiles

The program will decide automatically if partials should be included in the

refinement of the detector parameters and formation of the standard

profiles. If however you want to explicitly force the program to do so use

the following keywords: REFINEMENT INCLUDE PARTIALS for detector refinement

PROFILE PARTIALS for profile formation. To explicitly EXCLUDE partials from

profile formation use: PROFILE FULLS

9) Processing data from an offset detector (in two-theta)

To process data collected from an offset detector, the swing angle needs to

be specified, and the direct beam coordinates should be those corresponding

to a two-theta value of zero, unless the SWUNG_OUT subkeywords are given.

The areas for the standard profiles MUST be specified explicitly using the

PROFILE keyword.

TWOTHETA 15 BEAM SWUNG_OUT 99.96 124.5 ! Direct beam position on swung out !

detector PROFILE XLINES 0 50 100 150 200 YLINES 0 50 100 150 200

10) Excluding a "shadow" around the rim of the image

The LIMIT RSCAN keywords can be used to limit the "active" area of a

circular detector, or XSCAN and YSCAN for an rectangular detecor. All these

limits are measured from the physical centre of the detector.

eg. LIMITS RSCAN 146.5

11) Excluding more general "shadows"

The NULLPIX keyword can be used effectively to deal with more general

shadows, for example if a solid backstop support is used. All pixels with a

value less than that specified by the NULLPIX keyword will be treated as

lying outside the active area of the detector, and any spots containing

these pixels will be rejected.

In addition, rectangular areas of the detector can be excluded from spot

finding and integration by use of the LIMIT EXCLUDE keywords. See the

helpfile for details.

12) Resetting the saturation point of the detector

If there is evidence that the detector is saturating before the default

cutoff value, it can be reset with the OVERLOAD CUTOFF keywords:

eg OVERLOAD CUTOFF 75000

12) Limiting the resolution

Normally the resolution limit is set by the physical dimensions on the

detector. This can be overridden by the RESOLUTION keyword:

eg RESOLUTION 3.5

Both inner and outer resolution limits can be set:

eg RESOLUTION 20.0 3.5

The resolution can also be determined by the quality of the data, so that

the resolution limit is determined by the mean value of I/sig(I) within a

resolution shell.

eg RESOLUTION CUTOFF 5.0 |

----------------------------------------------------------------------------

Back to Contents Page

Appendix I - Changes in MOSFLM

Summary of major changes from version 5.30

Not yet summarised; see MAJOR CHANGES in Chapter 0.

----------------------------------------------------------------------------

Back to Contents Page

Appendix II Setting the measurement box parameters manually

IMPORTANT

The program now automatically optimises the measurement box parameters so

normally it is not necessary to optimise them manually. For difficult cases

the optimisation can be turned off (PROFILE NOOPTIMISE)and the manual

approach used.

Following 2 rounds of refinement, the program will display the average spot

profile for the spots used in the refinement. At this stage the user can

adjust the dimensions of the measurement box, and the average spot profile

will be recalculated. This can be done as many times as necessary. Note that

a value of zero must be entered as 99. When choosing the optimal box

parameters the following points should be kept in mind.

i) The accuracy of the weakest reflections will be determined by the

accuracy of determining the local background. There should therefore be at

least as many pixels in the background as in the peak (and preferably more,

although beyond a factor of 2 there is little additional gain). The program

has a sophisticated algorithm for rejecting pixels from the background if

neighbouring spots intrude into part of the background region, so this

should not necessarily enforce the use of a small measurement box. If

possible, however, enlarge the box in the direction of greatest spot

separation.

ii) Systematic errors in the integrated intensity will result if the spot

extends out of the peak region of the measurement box. This should be

avoided if at all possible.

iii) The inclusion of what are actually background pixels within the peak

region of the box will degrade the summation integration estimate of the

intensity (because they introduce only noise, no signal). However they will

have almost no effect on the profile fitted intensity, so there is very

little penalty involved. It is much more important to get all the spot

within the peak region than to keep background out of the peak region.

iv) The processing time is approximately directly proportional to the number

of pixels in the measurement box, so if this is a consideration, don't make

the box too large (but remember point (i) above).

v) The positional residual will increase as the peak size is increased, but

this really doesn't matter.

When you are happy with the measurement box parameters, just give a carriage

return to the query on changing the measurement box parameters, and the

program will go on to do the second round of positional refinement using

reflections from the entire detector (rather than just the centre). The

residual vectors are plotted as before. The program asks if you wish to

repeat this refinement. Even if you say No, it may repeat it anyway if the

residual is too high. It will then go on to integrate all the spots on the

image. As it does so, it will display the residual vectors between observed

and calculated positions for all fully recorded reflections above some

threshold intensity. Finally it gives some statistics on the measured

intensities, writes the intensities back to the generate file, and returns

the prompt:

==>

----------------------------------------------------------------------------

Back to Contents Page

Appendix III Overview of MOSFLM

1) Introduction

The MOSFLM suite of programs is designed to facilitate processing of

rotation data collected on either image plate or film. The suite originates

from the MOSCO system developed in Cambridge by Nyborg and Wonacott for use

on a PDP 11/10 (Nyborg & Wonacott, 1977) but it has been extensively

developed since that early version, primarily at Imperial College by A.J.

Wonacott, P. Brick and A.G.W. Leslie, and more recently at LMB. In

particular the much greater memory and cpu resources available in current

machines have been exploited (the first version ran on a machine with 28

Kbytes of memory, the current version uses 5 Mbytes of memory just to store

digitised images).

The basic procedure for data processing is independent of the type of

detector (film or image plate) although there are a number of useful

features which are only available for image plate data (particularly

automatic updating of cell parameters and crystal orientation).

The MOSFLM program

a) Overview.

MOSFLM performs the actual integration of the reflection intensities. It

generates the reflection list, reads the digitised image, integrates the

spots and writes the intensities and standard deviations into the generate

file and mtz file. The image plate version has the additional capability of

refining the crystal orientation and cell parameters during data processing

using the intensities of partially recorded reflections in the same manner

as the POSTCHK program. The program can be run interactively making use of

the graphical output options (X-windows of Tektronix emulation), which is

most useful when first characterising a new crystal or when dealing with

pathological cases.

Routine processing is generally done as a background job.

The normal operation of the program can be broken down into several steps as

outlined below. A summarised flow diagram is given in Figure 2.

ii) Generation of a reflection list for this image using the latest refined

values for crystal orientation, cell parameters and beam parameters. Crystal

orientation can be refined using a pattern matching procedure.)

iii) Location of diffraction spots in the central region of the image and

refinement of the detector parameters using the observed positions of these

spots. Determination of an average spot profile optimisation of measurement

box parameters.

iv) Location of diffraction spots in the outer regions of the detector and

further refinement of detector parameters.

v) The pixel values corresponding to the measurement boxes for all

reflections are extracted from the digitised image and written to a scratch

file. If addition of partially recorded reflections is being done (and this

is the default) then only partials at the end of the oscillation range are

chosen, and the pixel values for the other "half" of the partial on the NEXT

image are added to those of the current image before writing the pixel

values to the scratch file. Thus the partial addition is done within MOSFLM,

rather than at the AGROVATA stage. This is only valid if the effective

exposure time of both images is the same (eg the data is collected in the

"dose" mode rather than simply by time), and the origin (ie direct beam

position) and orientation of every image is identical.

vi)If post-refinement of the crystal orientation, cell parameters or beam

parameters is to be carried out, then the intensities of partially recorded

reflections occuring at the end of the oscillation range of the current

image and the start of the next image are evaluated as the measurement boxes

(for the current image only) are written to the scratch file in step (v)

(these intensities are evaluated by summation integration rather than

profile fitting). Providing that data over the required angular range is

available, post-refinement of the crystal parameters is then carried out.

vii) Steps (i) to (vi) are repeated for all the images to be used in forming

a set of standard profiles for evaluation of the reflection intensities.

Thus the scratch file will finally contain the measurement boxes of a number

of different images.

viii) The standard profiles are evaluated for several different regions of

the detector using the reflection data accumulated in the scratch file.

These profiles are then used to evaluate the reflection intensities for each

image, and the profile fitted and summation integration intensities and

standard deviations are written back to the generate file.

ix) Steps (i) to (viii) are repeated until all images have been processed.

A more detailed description of each of these steps follows.

---------------------------------|

|                            Read in first two images

|                   (the first image is the 'current image')

|  ------------------------------|-------------------------------------------|

|  |             Generate reflection list for current image                  |

|  |          using the latest parameters (orientation cell, etc)            |

|  |                             |                                           |

|  |       Refine detector parameters, initially using reflections from      |

|  |        the central region, then over the entire detector.               |

|  |        Optimise the measurement box parameters for the average          |

|  |        spot profile of centre of image. This is done for the first      |

|  |                image of every new BLOCK of data.                        |

|  |                             |                                           |

|  |                             |                                           |

|  |     Simultaneously integrate the two halves of partially recorded       |

|  |        reflections for use in post-refinement                           |

|  |                             |                                           |

|  |         Refine crystal orientation etc. using post-refinement           |

|  |                             |                                           |

|  |                 Are shifts acceptably small ?                           |

|  |                             |                                           |

|  |                             |----------YES---------------               |

|  |                             |                           |               |

|  |                             NO                          |               |

|  |                             |                           |               |

|  |            Is post-refinement in single image mode ?    |               |

|  |                             |                           |               |

|  |------------- YES -----------|                           |               |

|                                NO                          |               |

|                                |                           |               |

|              Is this the first post-refinement run ?       |               |

|                                |                           |               |

|--------------------YES---------|                           |               |

NO                          |               |

|                           |               |

Print warning message                 |               |

|----------------------------               |

Is refinement residual acceptably small ?                       |

|                                           |

STOP------------NO-------|                                           |

YES                                          |

|                                           |

Have sufficient images been accumulated to form standard profiles ?         |

|                                           |

|-------------- NO --- Read next image -----

YES

|

Rewind scratch file. Form standard profiles.

Optimise measurement box parameters for each profile.

Reject all background pixels overlapped by neighbouring spots.

Integrate all images in this block

Write intensities back to generate file.

Apply Lp corrections, reduce indices to asymmetric unit write to MTZ file.

Calculate R-factor for symmetry related reflections.

Write summary file information.

|

Process next block of images (if any)

Figure 2. A Flow diagram of the operation of MOSFLM when using the post-

refinement option.

c) Generation of the reflection list

.

Generation of the reflection list is performed using the Reeke algorithm.

d) Initial refinement of detector parameters.

The first step in the refinement of the detector parameters is the location

of up to 60 suitable reflections. Generally, only fully recorded reflections

are selected, as the position of the centroid of a partially recorded

reflection will depend on its degree of partiality. For crystals with a high

mosaic spread, there may be too few fully recorded reflections to allow a

satisfactory refinement, so partially recorded reflections may have to be

used. Similarly, overloaded reflections will have a poorly determined centre

of gravity, although for very intense images it may be necessary to include

some overloaded reflections.

The refined parameters are: i) The crystal to detector distance (XTOFD). ii)

The position of the centre of the diffraction pattern (XCEN,YCEN). iii) A

relative scale factor applied to the Y coordinates (YSCALE). iv) Small

rotations of the detector about a horizontal axis (TILT) and a vertical axis

(TWIST) v) Rotation of the detector about the X-ray beam direction (OMEGA)

vi) Radial (ROFF) and tangential offsets (TOFF) (Mar scanners only, see

below) vii) ONLY if explicitly requested (REFINEMENT FREE keywords), the

amplidue of the radially dependent radial and tangential offsets RDROFF,

RDTOFF in mm (Mar scanners only).

Further details of the refinement procedure are given in Appendix IV.

Refinement is carried out for a fixed number of cycles, and is followed by a

display of the average spot profile of the reflections used in refinement

(but not partials) if requested. If the program is being run interactively

and automatic measurement box optimisation is NOT being performed, the user

has the option to update the parameters defining the measurement box at this

stage.

If insufficient reflections are found for refinement (ie less than 20), then

the program will automatically attempt to find additional reflections in a

number of different ways. If the majority of reflections are too weak, then

the threshold will be reduced (but only dwon to one sigma). If the majority

of reflections are overloaded, it will allow the use of overloaded

reflections. If there are fewer than 20 non-overloaded reflections in the

central region of the detector, then the size of this region will be

enlarged by 10mm (in X and Y). If the final residual is greater than a

preset limit, processing will be abandoned.

e) Location of reflections in outer region of the detector and further

refinement.

Following successful refinement using reflections from the inner region of

the detector, a list of all fully recorded reflections ouside this central

region is prepared (partials will be included if requested). After sorting

this list on the detector X coordinate, it is divided into 8 bins with an

equal number of reflections in each bin. Within each bin, reflections with

an I/sigma(I) value greater than a preset limit are chosen until a maximum

of 5 reflections have been found. If less than 30 reflections are found in

total, the I/sigma(I) cutoff is automatically reduced and the search

repeated (to a minimum I/sigma(I) cutoff value of 2). These reflections are

combined with 20 reflections selected from the central region for final

refinement of the detector parameters. The final rms positional residual

(given in mm) will depend on the spot size, the strength of the image and

the accuracy of the cell parameters, but typical values are 0.02-0.03mm for

a reasonably strong image and up to 0.07-0.08mm for relatively weak images.

Larger values suggest an error in cell parameters, which can usually be

corrected using the post-refinement option. If partial reflections are

included in the refinement the positional residual will be significantly

larger (eg 0.06-0.07mm for a strong image, 0.10-0.15mm for a weak image).

g) Extracting the measurement boxes from the digitised image.

A final list of all reflections to be integrated is prepared and sorted on

the detector X coordinate. Using a circular buffer and working through the

digitised image stripe by stripe, the pixel values corresponding to the

measurement boxes of these reflections are accumulated and written to a

scratch file. The size of the peak area of the measurement box is expanded

automatically (by 2 pixels at a time to maintain an odd number of pixels) in

order to allow for the increase in spot size due to obliquity of incidence

of the diffracted beam on the detector. This is a function of the spot size

(determined by collimation and mosaic spread of the crystal), the effective

detector thickness and the Bragg angle. If post-refinement is to be carried

out (image plate data only), then for each reflection that is partially

recorded at the end of the oscillation range of the current image, the

identical pixels on the next image in the series are also accumulated. This

requires both images to be stored in memory. This is only practical if the

detector has a fixed origin and orientation (eg the Mar Research IP scanner)

so that the predicted position of the partially recorded reflection is

identical in the digitised image on both images. It is unclear at present

whether images scanned using an off-line image plate scanner will give

sufficiently reproducible results for the method to be applicable. The

measurement boxes for the two halves of the partial are then integrated

(using summation integration) to give the intensity of the two components,

which in turn gives an observed degree of partiality:

P(obs) = (Icurr/(Icurr+Inext))

where Icurr is the intensity on the current image and Inext is the intensity

on the next image. In order to minimise the errors due to the finite pixel

size, the peak area of each measurement box is interpolated onto a grid

centred on the calculated position of the reflection, using linear

interpolation. If this is not done then the standard profiles, which are

formed by averaging all fully recorded reflections within a limited area of

the detector, will be artificially broadened because the true reflection

position can be up to half a pixel away from the centre of the measurement

box (in any direction). The interpolation procedure significantly improves

the fit of the profiles, particularly for image plate data, although in this

case the linear interpolation procedure is not ideal in view of the very

large dynamic range, which can lead to local gradients of 20,000 counts per

pixel for strong reflections. The summation integration intensity is

unaffected by the interpolation.

h) Post-refinement of crystal orientation, cell parameters and beam

parameters.

The degree of partiality of a partially recorded reflection is a function of

the crystal orientation, cell parameters and beam parameters (mosaic spread

and horizontal and vertical divergence of the X-ray beam). These parameters

can therefore be refined using the observations P(obs) described in section

(g) if there is a model for the rocking curve, that is, the relationship

between the distance of a reciprocal lattice point from the Ewald sphere at

the end of the rotation and the resulting intensity of the reflection on

that image expressed as a fraction of the total intensity. This rocking

curve is therefore used to convert the observed partiality P(obs) to the

position of the reciprocal lattice point at the end point of the rotation.

It is this conversion which is a function of the mosaic spread and beam

divergences. The position of the reciprocal lattice point is then a function

of the crystal orientation and cell parameters. It should be noted that this

is independent of the reflection coordinates on the detector or the crystal

to detector distance.

When data from a single pair of images is used to refine crystal

orientation, there is essentially no information on the orientation of the

crystal around the X-ray beam, and only the missetting angles around the

rotation axis (PSIZ) and the direction normal to the rotation axis and X-ray

beam (PSIY) are refined. Following refinement these missetting angles are

converted back to the standard missetting angles PHIX,PHIY,PHIZ which

describe the crystal orientation at phi=0. In general, if the crystal

symmetry is lower than trigonal, not all the cell parameters will be well

defined using data from one pair of images (the spacing along the X-ray beam

direction is poorly defined) and in such cases it is advisable to use an

angular wedge of data (typically 5-10 degrees in phi) accumulated over a

number of successive images. Even then, some cell parameters may not be well

defined (large standard deviations and shifts) and in such cases individual

cell parameters can be fixed if desired. However it is preferable to use the

POSTREF SEGMENT option to get an accuate cell in advance of the processing

run in these cases.

When an angular wedge of data is to be used, cell parameter refinemnt is

delayed until sufficient images have been processed (the crystal orientation

is still refined after evry image). After the first refinement, if there are

large shifts in any of the refined parameters the processing will be

restarted at the first image using the updated parameter values. This can be

repeated a number of times, allowing a reasonably large radius of

convergence from inaccurate starting parameters. Post-refinement is then

carried out after every image, by deleting the data from the image with the

smallest phi value and adding in the data from the latest image. When a

single pair of images is used for the refinement, then a large shift in

missetting angles or cell parameters will result in the reprocessing of that

image.

The post-refinement procedure is, not suprisingly, most successful with

reasonably strong, high resolution, data. For weak data at low resolution,

the correlation between different parameters in the refinement can lead to

instability which manifests itself as unrealistically large variations in

cell parameters and missetting angles. For example, in the case of

monoclinic data collected by rotating about the unique axis, there is a

strong correlation between the monoclinic beta angle, the missetting angle

around the rotation axis, phiz, and the relative values of the a and c cell

parameters. In these cases data from a number of "blocks" of images at

different phi values should be used to obtain accurate cell parameters prior

to processing, and the cell parameters should be kept fixed during the

processing (POSTREF SEGMENT option)

i) Formation of the standard profiles.

Normally a number of different standard profiles will be determined for

different areas on the detector. These areas are by definition rectangular,

and are defined by a set of lines parallel to the detector X and Y

directions.

By default for high resolution data (greater than 2.5A) the program will

generate 6 lines in each direction, giving 5*5=25 standard profile areas of

which 4 will be outside the circular resolution limit, leaving 21 active

profiles. For resolutions below 2.5A it will generate 4 lines giving 3*3=9

standard profiles.However the user may define his own areas on the detector

by supplying the coordinates (in mm) of the defining lines (XLINES,YLINES).

The overall measurement box size for each of the standard profiles will be

determined by expanding the peak region of the measurement box determined

for the central region of the detector to allow for the increase in spot

size due to obliquity (this depends on the Bragg angle and the nominal

detector thickness). However the optimal background parameters (NRX,NRY,NC)

will be determined separately for each of the standard profiles.

The standard profiles themselves are determined by summing all fully

recorded reflections above a certain threshold intensity. When measuring

data from crystals with a very large mosaic spread, there may be very few

fully recorded reflections on each image. In these circumstances there is

the option to add together the two components of a partial at the stage when

the measurement boxes are written to the scratch file (see Fig. 2). In this

way reflections which are actually partially recorded at the end of the

rotation range of the current image will be treated as if they were fully

recorded on the current image, and in particular these reflections are used

in forming the standard profiles. This procedure depends on successive

images having the same exposure time and incident flux. This threshold is

defined in terms of the average, background subtracted, peak pixel value

(over 9 central pixels) exceeding the rms variation in background by a

specified factor, usually 2. A best least-squares background plane is fitted

to the summed reflections (see below for details of background outlier

rejection). The resulting pixel values, after scaling the central pixel

value to 10,000, is taken as the standard profile. Two criteria are applied

to determine whether the resulting profile is satisfactory. First, there is

a minimum acceptable number of reflections that contribute to the profile

(10) and secondly the rms variation in the background plane (after scaling)

must not exceed a specified limit. If a particular profile fails either

test, then it is improved by adding in the summed reflections from adjacent

regions on the detector. This will normally produce an acceptable profile,

but if it does not then no profile fitting will be attempted for that region

of the detector. Clearly the accuracy of a given profile will depend on the

number (and intensity) of the reflections contributing to that profile. The

program therefore allows the standard profiles to be accumulated over a

number of successive images, rather than forming them on an image by image

basis. Typically, the profiles will be accumulated over between 5 and 10

images. This significantly improves the signal to noise in the profiles, but

it would be unwise to average over too many images as the profiles

themselves may vary slightly with rotation angle as a result of changes in

the diffracting volume of the crystal or radiation damage, which can lead to

an increase in mosaic spread an hence spot size.

j) Reflection integration.

Following formation of the standard profiles, the reflections on all images

contributing to the profiles are integrated. The first step in this

procedure is fitting the best background plane to the background pixels in

the measurement mask. In order to deal with outliers (due to cosmic rays,

spots from a satellite crystal etc) in the background region of the

reflection, the initial background plane is determined using a fraction

(between 0.5 and 1.0) of the total number of background points, selecting

those pixels with the lowest values. The constant component of the

background plane is then adjusted to correct for the systematic bias

introduced by selecting the lowest pixel values (assuming a Gaussian

distribution). This plane is then used to reject outliers, which are defined

as pixels with values which deviate from the plane by more than a fixed

number (usually 3) standard deviations, where the standard deviation is

based on Poissonian counting statistics. The same procedure is applied to

determining the centre of gravity of spots used in positional refinement

(sections d and e). The profile fitted intensities and standard deviations

are evaluated using weighted profile fitting and methods based on those

originally described by Rossmann (1978). For every reflection, a new profile

is evaluated by linear interpolation of the standard profiles of the regions

surrounding that reflection. The interpolation is based on the distance of

the reflection from each standard profile, where the coordinates of the

profiles are calculated as the intensity weighted mean of all the

reflections contributing to that profile. For most reflections, four

different profiles will contribute to the variable profile, but for

reflections near the periphery of the detector there may be only three or

two contributing profiles, while no sensible interpolation can be done for

the outermost reflections. This procedure provides a more accurate modelling

of the way in which the profile varies across the face of the detector.

The summation integration intensity and standard deviation is also evaluated

for every reflection, and both profile fitted and summation integration

intensities and standard deviations are written back to the generate and mtz

files.

Individual reflections are flagged as 'badspots' (which are rejected by

MOSFLM) if they fail any of the following tests: 1) The rms fit of the

background plane must not exceed a factor (3) times the variation expected

on the basis of counting statistics. 2) The intensity should not be negative

with an absolute value greater than 5 standard deviations. 3) The fit of the

profile should not be worse than a factor (3) times the expected fit based

on counting statistics. If a reflection fails this test, only the profile

fitted intensity (not the summation integration intensity) will be rejected.

4) The background plane gradient must not exceed a preset value (by default,

spots with a ratio of the either background plane gradient (a or b) to the

average background (ie a/c or b/c) greater than 0.03 wil be rejected.

5) The reflection must not contain more than a specified number of saturated

pixels (ie be overloaded). The intensities of these reflections can be

estimated by profile fitting if requested, using only that part of the peak

which is not saturated in fitting the standard profile. Typically only a

handfull of reflections (between 0 and 5) will be rejected on any image.

Larger numbers are indicative of problems in processing, eg poor profile fit

due to inaccurate cell parameters.

ii) Orientation refinement by pattern matching.

This option permits the refinement of the crystal orientation by

automatically optimising the fit between the predicted and observed

patterns, by maximising the predicted intensity (ie finding the orientation

for which the sum of the intensities of the predicted reflections is a

maximum). The basic algorithm is identical to the 'convolution' procedure

described by Rossmann (1978). There are minor differences, the most

significant being:

a) The total predicted intensity is calculated while stepping the missetting

angle (either PSIY or PSIZ) through a fixed number of angular steps

(typically 9) centred on the current value of that angle. The best value for

that parameter is then chosen as the midpoint of the two steps at which the

intensity has dropped by 15 standard deviations from its value at the centre

of the range (ie the current value). This is in contrast to Rossmann's

procedure which simply chooses the value at which the intensity is at a

maximum. This makes the refinement less susceptible to errors introduced by

statistical fluctuations in the total summed intensity. The step size is

then multiplied by a damping factor (normally 0.25) and the grid search

repeated.

b) When generating the reflection list for different values of PSIZ (say)

with a view to optimising the summed intensity, the beam divergence in the

horizontal direction is increased by an amount equal to the largest step

applied to PSIZ, so that the search is insensitive to errors in PSIY.

Similarly, when PSIY is being refined, the vertical divergence is increased

by the same amount.

With these modifications, the algorithm will converge from an initial

orientation up to 2-3 degrees in all three missetting angles for problems

with cell parameters between 50 and 100A. However, the advent of

autoindexing algorithms has made this option essentially obsolete.

----------------------------------------------------------------------------

Back to Contents Page

Appendix IV Coordinate systems used by MOSFLM

a)     Detector  Coordinates

b)     Camera Constants

c)     Coordinate Transformations

References

----------------------------------------------------------------------------

a) Detector Coordinates

The coordinates systems used by MOSFLM are illustrated in Fig. 3. The

crystal orientation is defined relative to the camera coordinate frame

(X,Y,Z), with X along the X-ray beam (away from the source), Z along the

rotation axis, and Y forming a right-handed set. A positive phi rotation is

anticlockwise viewed from +Z looking towards the origin. The orientation

matrices (A and U) are defined relative to this axial system.

Coordinates of reflections are calculated in the detector frame (Xd, Yd),

which has its origin at the point of intersection of the direct beam and the

detector, with Yd parallel to the camera Z axis and Xd parallel to the

camera Y axis.

Finally, the pixels of the digitised image have coordinates (Xs, Ys). For

image plate data and films which have been scanned on a microdensitometer in

the normal orientation (that is, with Xd around the circumference of the

scanner drum, Yd along the axis of the drum), the origin of this coordinate

frame is the lower left corner of the image viewed from behind the detector

looking towards the X-ray source (Fig. 3). Thus the scanner axis Xs is

(ideally) antiparallel to the detector axis Yd, while scanner axis Ys is

(ideally) parallel to detector axis Xd.

For film data, there is the option to scan the film in an orientation

corresponding to a 90 degree clockwise rotation from that shown in figure 3,

in which case Xs is (ideally) parallel to Xd and Ys is (ideally) parallel to

Yd.

The coordinates of the direct beam position in the scanner coordinate frame

(Xs, Ys) are denoted XCEN, YCEN. If the angle between Xd and Xs is denoted

by OMEGA, then for image plate data the ideal value of OMEGA is 90 degrees.

For film data scanned in the normal orientation (as in Fig. 3), OMEGA may

deviate slightly from 90 degrees as its precise value will depend on how the

film is placed on the scanner. For film scanned in the alternative

orientation, OMEGA will be close to zero.

/!

Y-axis                     / !

^                       /  !

!                      /   !

!                     /    !

!   /                /  Xd !

!  /                / * ^  !

! /                 ! 3 !  !

!/      X-ray beam  !   !  !

/-----------------------/--!---->X-axis

/                    !  / *1!

<-/-                    ! /    !

/  \+ve phi            ! Yd  /

/   /                   ! 2  /

/                        ! * /

Z-axis                 Ys ^ _/

Rotation                    ! /| Xs

axis                       !/

O

Figure. 3

The coordinate systems used by MOSFLM. X,Y,Z are an orthogonal frame centred

at the point of intersection of the X-ray beam and the rotation axis.

Positive phi rotation is anticlockwise as viewed down the Z axis towards the

origin (as indicated). The ideal detector coordinate frame (Xd,Yd) has its

origin at the point of intersection of the detector plane and the X-ray

beam. Yd is parallel to the rotation axis and Xd orthogonal to it. The

scanner coordinate frame (Xs,Ys) has its origin at the lower left corner of

the image (camera-mans view, ie looking towards the source). If necessary

(as it is for images from the Mar scanner) the image will be inverted left

to right when read by the program so that internal to MOSFLM the first pixel

in the digitised image is in this corner. However this is entirely hidden

from the user, and in the display, for example, the first pixel in the image

(with pixel coordinates 1,1) will be in the lower right corner of the

displayed image.

Ys is then the most rapidly changing direction in the digitised image, Xs

the more slowly changing direction.

b) Camera Constants

i) Image plate data

The coordinates of the direct beam position XCEN, YCEN in the scanner

coordinate frame must be supplied by the user. Any deviation in the refined

position of the centre of the diffraction pattern from these coordinates are

denoted by the camera constants CCX, CCY. It should be noted that CCX, CCY

are defined in the detector frame (Xd, Yd), NOT in the scanner coordinate

frame. This a positive CCX represents a displacement along +Ys, while a

positive CCY is a displacement along -Xs. Any deviation of the angle OMEGA

from its expected value of 90 degrees is referred to by the camera constant

CCOMEGA. The camera constants allow for errors in the user defined position

of the direct beam (CCX and CCY) and in the alignment of the scan direction

of the image plate relative to the camera (and detector) axes (CCOMEGA).

ii) Film data

For film data the camera constants CCX, CCY denote the deviation of the

refined position of the centre of the diffraction pattern from the midpoint

of fiducial marks 1 and 3 (Fig. 3), as appropriate for an Enraf Nonius

oscillation camera. They are expressed in the detector frame (Xd, Yd), so

that they are independent of the orientation of the film on the scanner. The

line joining fiducial marks 2 and 3 is assumed to be parallel to the

detector axis Xd , and any deviation is denoted by the camera constant

CCOMEGA. Note that for film data, the value of OMEGA will not necessarily be

90 degrees (normal orientation) or 0 degrees (rotated orientation) because

it will depend on exactly how the film is placed on the scanner. The

orientation of the film on the scanner does NOT affect the relative

orientation of the line joining fiducials 2 and 3 and the detector axis Xd

at the time of collecting the data. The camera constants allow for errors in

the positioning of the cassette on the carousel and misalignment of the

camera itself.

c) Coordinate Transformations

The basic transformation between the calculated detector coordinates of the

reflections (Xd, Yd) and the scanner coordinates in the digitised image (Xs,

Ys) is given by:

Xs = XCEN + XTOFRA*( Xd Cos(OMEGA) - Yd Sin(OMEGA))

Ys = YCEN + XTOFRA*YSCALE*( Xd Sin(OMEGA) + Yd Cos(OMEGA))

where:

XCEN, YCEN are the coordinates of the centre of the diffraction pattern (the

direct beam position) in the scanner coordinate frame.

XTOFRA is a refined scale factor which accounts for variations in crystal to

detector distance. Its nominal value is 1.0, as the actual crystal to

detector distance is implicit in the detector coordinates.

YSCALE is a further refined scale factor defining the relative scales along

the Ys and Xs axes. For image plate data YSCALE should be unity, but for

film data the pixel size around the circumference of the drum is increased

slightly due to the finite thickness of the X-ray film (by about 0.2%), so

YSCALE should have a value close to 0.998.

OMEGA is the (refined) angle between Xd and Xs .

For image plate data, an additional four parameters have been introduced to

allow for errors in the alignment of the camera and distortions introduced

by mechanical misalignment of the scanner. TILT and TWIST allow for the

detector plane being non-normal to the incident X-ray beam, and correspond

to a rotation about an axis parallel to the camera Z axis and camera Y axis

respectively (ie horizontal and vertical axes in the conventional

arrangement). The effect of these rotations on spot positions can be

adequately modelled (for alignment errors of a few degrees or less) by a

variation in crystal to detector distance that is dependent on Xd (TILT) or

Yd (TWIST). Thus XTOFRA is replaced by:

XTOFRA' = XTOFRA + TILT*Xd + TWIST*Yd

When expressed in this way, TILT and TWIST when divided by the crystal to

detector distance are the actual rotations (in radians). The refined values

printed by the program are expressed in hundredths of a degree.

For the Mar Research image plate scanner, it is assumed that the first pixel

of the spiral scan (at the outer edge of the image plate) is at a known

radius from the centre of the image plate. Any error in the presumed radius

of this first pixel will result in a (fixed) radial offset in the positions

of all reflections. This is modelled by a refineable radial offset (ROFF). A

second assumption is that the radial scan is indeed exactly radial, ie would

pass through the centre of rotation of the image plate. If this is not the

case, to a good approximation it will result in a (fixed) tangential offset

in the position of all reflections (TOFF). (Note that this assumption would

break down for very small radii, but as the inner radial limit of the scan

is 10mm this is not a problem in practice).

It has been found that ROFF and TOFF show a radial dependence that is

significant in some 30cm Mar scanners.

This has been modelled as a sine-wave dependence, giving:

ROFFTOT = ROFF + RDROFF*SIN(PI*R/RSCAN) TOFFTOT = TOFF +

RDTOFF*SIN(PI*R/RSCAN)

Where RSCAN is the scanner radius and R is defined below.

Defining:

Xm = Xd Cos(OMEGA) - Yd Sin(OMEGA)

Ym = Xd Sin(OMEGA) + Yd Cos(OMEGA)

PSI = atan (Ym/Xm)

then the transformation, including the distortion components, is given by:

Xs = XCEN + (XTOFRA + TILT*Xd + TWIST*Yd)*Xm + ROFFTOT*Cos(PSI) -

TOFFTOT*Sin(PSI)

Ys = YCEN + (XTOFRA + TILT*Xd + TWIST*Yd)*YSCALE*Ym + ROFFTOT*Sin(PSI) +

TOFFTOT*Cos(PSI)

These equations are used in the least squares refinement of the detector and

distortion parameters, using the measured centres of gravity of the selected

refinement spots (in the scanner coordinate frame) as the observed

positions.

For film data, three distortion parameters are refined. The first two, TILT

and TWIST are identical to those used for image plates. The third parameter

is to allow for bulging of the X-ray film in the cassette. This is modelled

as a radially dependant variation in the crystal to detector distance,

corresponding to a conical distortion which is radially symmetric.

Using Xm and Ym as defined above, and defining in addition:

R = SQRT( Xd*Xd + Yd*Yd)

the transformation for film data becomes:

Xs = XCEN + (XTOFRA + TILT*Xd + TWIST*Yd + BULGE*R)*Xm

Ys = YCEN + (XTOFRA + TILT*Xd + TWIST*Yd + BULGE*R)*YSCALE*Ym

Refinement procedes in an identical manner to image plate data, with the

exception that as the BULGE parameter is poorly defined when using only data

from the central area of the image, it is held fixed at its input value

during the initial refinement.

The refinement of crystal to detector distance (as XTOFRA) and the YSCALE

parameter allow for errors in crystal cell parameters to be at least

partially compensated when predicting reflection positions. This can be

important in cases where no accurate values are initially available, as the

post-refinement option for refining cell parameters is of course only

available once the images have been measured.

References

A. Jones, K. Bartels & P. Schwager (1977) in 'The Rotation Method in

Crystallography', U.W. Arndt & A.J. Wonacott, eds, North Holland Publishing

Co.

W. Kabsch (1988) J. Appl. Cryst., 21, 67-71.

W. Kabsch (1993) J. Appl. Cryst., 26, 795-800.

I. Steller, R. Bolotovsky & M.G. Rossmann (1998) J. Appl. Cryst. 30,

1036-1040

A.G.W. Leslie (1990) in 'Crystallographic Computing', Oxford University

Press.

J. Nyborg & A.J. Wonacott (1977) in 'The Rotation Method in

Crystallography', U.W. Arndt & A.J. Wonacott, eds, North Holland Publishing

Co.

M.G. Rossmann, A.G.W. Leslie, S. Abdel-Meguid & T. Tsukihara (1979), J.

Appl. Cryst., 12, 570-581.

M.G. Rossmann (1979), J. Appl. Cryst. 12, 225-238.

F.K. Winkler, C.E. Schutt & S.C. Harrison (1979), Acta Cryst. A35, 901-911.

A.J. Wonacott (1977) in 'The Rotation Method in Crystallography', U.W. Arndt

& A.J. Wonacott, eds, North Holland Publishing Co. |

----------------------------------------------------------------------------

Harry Powell

Last modified: Wed Jul 7 17:24:05 BST 1999